ME1000 RF Circuit Design

Tutorial 3

Impedance Transformation and Impedance Matching and Microwave Filter

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

## Question 1

Using 2 lumped-elements analytical method, match a load consisting of a 19.8 nH inductance in series with a 75 resistance to a voltage source, with source impedance of 50.  Calculate the nodal Q factor of the matching network and estimate the matching bandwidth.  Assume an operating frequency at 500 MHz.

## Question 2

Using 2 lumped-elements analytical method, match a load consisting of a 10.0 pF capacitance in parallel, with a 120 resistance to a voltage source with source impedance of 300 at 800 MHz.   Calculate the nodal Q factor and estimate the matching bandwidth.

## Question 3

With the aid of the Smith Chart, design a single-stub transformation network to match a complex load consisting of a 4.0 pF capacitance in parallel with a 120 resistance to a voltage source, with source impedance of 200 at 1900 MHz.  Assuming the transmission line used has Zc = 50, phase velocity vp=0.7C, where C is the speed of light in vacum, C = 2.998108 m/s.

Question 4 (Extra)

Repeat Question 3 using a double–stub transformation network, design the network for nodal Q factor equal or greater than 3.

Question 5 (Extra)

Repeat Question 4, using a hybrid double-stub transformation network.  What is the advantage of using a double-stub network as compared to a single-stub network?

## Question 6

Design a single-section quarter wave transformer to match a complex load consisting of a 2.7 pF capacitance in parallel with a 120 resistance at 6 GHz to a 50 characteristic impedance transmission line.  Suppose the system is to be implemented on a piece of printed circuit board (PCB) with r = 8.0 (alumina substrate) and thickness = 1.0 mm, sketch the layout of the transmission lines, assuming the resistance and capacitance to be surface mounted components.

Question 7

- Design a 3rd order LC Chevbyshev bandpass filter (BPF) with the following parameters: Center frequency = 868.30 MHz, 3dB bandwidth = 78 MHz, ripple factor = 0.5dB.  In your solution show the low-pass prototype schematic, frequency transformation and impedance scaling procedures, and the final schematic.

- If you were to implement the BPF schematic on a double-sided printed circuit board (PCB), with the surface-mounted components on the top side, sketch the layout.   Also list out the precautions taken in coming up with the PCB layout.

- What do you think will happen to the values of the LC elements of your BPF, if we gradually reduce the ripple factor of the Chevbyshev BPF to 0 dB?