ME1000 RF Circuit Design

Tutorial 4

3-Port and 4-Port Microwave Components

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

## Question 1

Write down the S matrix of a circulator.  Using the S matrix, explain the function of the circulator.  What are the conditions imposed on the S matrix of the circulator in order for it to be realizable?

Question 2

List down 3 types of power divider configuration, briefly explain how these dividers work.  Also compare the pros and cons of each power divider.

Question 3

Design a microstrip equal-split Wilkinson power divider matched to impedance of 50Ω at all its ports.  The dielectric constant r of the printed circuit board (PCB) is 2.7, thickness of the board 1.0 mm and the required operating frequency is 3.1 GHz.  Sketch the layout of the PCB (just the top side will suffice).

Question 4

Explain the characteristics of an ideal directional coupler.  Elaborate on the definitions of coupling factor, directivity and isolation.