ME1000 RF Circuit Design

Tutorial 3

Impedance Transformation and Impedance Matching and Microwave Filter

(Solution)

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

## Question 1

Using 2 lumped-elements analytical method, match a load consisting of a 19.8 nH inductance in series with a 75 resistance to a voltage source, with source impedance of 50.  Calculate the nodal Q factor of the matching network and estimate the matching bandwidth.  Assume an operating frequency at 500 MHz.

Solution:

The calculation using MatchCAD worksheet is shown below:

Load impedance declarations

Target impedance declaration

Operating frequency

Finding the X and B

Finding the elements to synthesize X and B

## Question 2

Using 2 lumped-elements analytical method, match a load consisting of a 10.0 pF capacitance in parallel, with a 120 resistance to a voltage source with source impedance of 300 at 800 MHz.   Calculate the nodal Q factor and estimate the matching bandwidth.

Solution:

The calculation using MatchCAD worksheet is shown below:

## Question 3

With the aid of the Smith Chart, design a single-stub transformation network to match a complex load consisting of a 4.0 pF capacitance in parallel with a 120 resistance to a voltage source, with source impedance of 200 at 1900 MHz.  Assuming the transmission line used has Zc = 50, phase velocity vp=0.7C, where C is the speed of light in vacum, C = 2.998108 m/s.

Solution:

The calculation using MatchCAD worksheet and the fkSmith software is shown below:

Question 4 (Extra)

Repeat Question 3 using a double–stub transformation network, design the network for nodal Q factor equal or greater than 3.

Solution:

Implement the stubs using transmission line.  For B2 use a shorted transmission line section.  For B1 use an opened transmission line section.  For both stubs, assume  = 56.886 as in Question 3.

Question 5 (Extra)

Repeat Question 4, using a hybrid double-stub transformation network.  What is the advantage of using a double-stub network as compared to a single-stub network?

Solution:

Implement the stubs using lumped elements.  For B2 use an inductor while for B1 use a capacitor.

## Question 6

Design a single-section quarter wave transformer to match a complex load consisting of a 2.7 pF capacitance in parallel with a 120 resistance at 6 GHz to a 50 characteristic impedance transmission line.  Suppose the system is to be implemented on a piece of printed circuit board (PCB) with r = 8.0 (alumina substrate) and thickness = 1.0 mm, sketch the layout of the transmission lines, assuming the resistance and capacitance to be surface mounted components.

Solution:

Implementing the quarter-wave microstrip line:

Top view of the layout

4.6 mm

19.0 mm

To parallel

RC load

Question 7

- Design a 3rd order LC Chevbyshev bandpass filter (BPF) with the following parameters: Center frequency = 868.30 MHz, 3dB bandwidth = 78 MHz, ripple factor = 0.5dB.  In your solution show the low-pass prototype schematic, frequency transformation and impedance scaling procedures, and the final schematic.

Solution:

Please refer to the lecture notes for the synthesis procedure using the Insertion Loss Method.  Use the table for Chevbyshev Low-pass Prototype to obtain the basic LPP schematics, then perform frequency scaling and impedance denormalization to get the final schematic.

- If you were to implement the BPF schematic on a double-sided printed circuit board (PCB), with the surface-mounted components on the top side, sketch the layout.   Also list out the precautions taken in coming up with the PCB layout.

Solution:

Some precautions:

(1) Keep all traces between components footprint as short as possible, since we didn’t include the trace in our schematics.

(2) Keep the solder pads as small as possible (as permitted by the soldering process).

(3) Put parallel SMT inductor and capacitor on opposite sites, to prevent EMI between components.

(4) Use ample plated through-hole to ground the components.

- What do you think will happen to the values of the LC elements of your BPF, if we gradually reduce the ripple factor of the Chevbyshev BPF to 0 dB?

Solution:

This is something interesting, it has to do with polynomials of Chebyshev and Butterworth.  What you will see is that if you insist that ripple factor be 0 dB, the filter’s frequency response actually approaches Butterworth type!  This is just another manifestation of the conservative law of Nature.