ME1000 RF Circuit Design

Tutorial 4

3-Port and 4-Port Microwave Components

(Solution)

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

## Question 1

Write down the S matrix of a circulator.  Using the S matrix, explain the function of the circulator.  What are the conditions imposed on the S matrix of the circulator in order for it to be realizable?

Solution:

SCW=001100010

Port 1

Port 2

Port 3

Consider an incident vector of the form:	a=a100

The the scattered vector from the circulator’s terminals is:	b=001100010a100=0a10

We observe that the incident energy into port 1 is diverted to port 2, also port 1 is matched.

Consider another incident vector of the form:	a=0a20

The scattered vector is:	b=0011000100a20=00a2

Again we observe that incident energy into port 2 is diverted to port 3.  It can be easily shown that incident energy into port 3 will be diverted to port 1.  This device is called a clockwise circulator.  A dual of this device is the anti-clockwise circulator, with its scattering matrix given by:

SACW=010001100

For the circulator to be realizable, we note that it’s scattering matrix is matched, lossless (try to prove this) and non-reciprocal.

Question 2

List down 3 types of power divider configuration, briefly explain how these dividers work.  Also compare the pros and cons of each power divider.

Solution:

- Circlulator.

- T-junction power divider using transmission lines junction.

- Resistor divider.

- Wilkinson power divider.

Question 3

Design a microstrip equal-split Wilkinson power divider matched to impedance of 50Ω at all its ports.  The dielectric constant r of the printed circuit board (PCB) is 2.7, thickness of the board 1.0 mm and the required operating frequency is 3.1 GHz.  Sketch the layout of the PCB (just the top side will suffice).

Solution:

Zc = 50

2⋅Zc=70.71

For r = 2.7 and d = 1.0 mm and using microstrip line structure:

1) For Zc = 50, W/d = 2.72, W = 2.72 mm, eff = 2.215.

2) For Zc = 70.71, W/d = 1.53, W = 1.53 mm, eff = 2.136.  Furthermore for this Tline:

vp=1εeffεoμo=2.0513×108λ=vpf=2.0513×1083.0×109=0.0684=68.4mmλ4=17.1mm

We would like to implement the 70.71Ω Tline as semi-circle, it’s radius will be: r=17.1π=5.44mm.

Question 4

Explain the characteristics of an ideal directional coupler.  Elaborate on the definitions of coupling factor, directivity and isolation.

Solution:

Directional coupler:

When Port 4 is terminated with its respective impedance and a voltage wave is injected into Port 1, a small amount of energy will be coupled to Port 3, while the remaining incident energy will go to port 2.  Port 3 is called the ‘Coupled’ port while Port 2 is usually called the ‘Through’ port.  A properly designed directional coupler is lossless, so energy/power coming out of Port 2 and 3 equals the incident energy into Port 1.  Moreover no energy should leaks from Port 4, which is aptly named the ‘Isolated’ port.  A typical scattering matrix for the directional coupler is given by:

S=0αjβ0α00jβjβ00α0jβα0	or	S=0αβ0α00−ββ00α0−βα0

For instance let the incident vector be:	a=a1000

Then	b=0αjβ0α00jβjβ00α0jβα0a1000=0αa1jβa10

Since incident power P1 is: P1=12a12

Through power:	P2=12αa12=12α2a12

Coupled power:	P3=12jβa12=12β2a12

From the 3 power reading, we could define the following quantities to describe the performance of the Directional Coupler:

Coupling Factor =10log10P1P3=10log101β2=−20log10β dB

Directivity =10log10P3P4=−20log10βs14 dB

Isolation =10log10P1P4=−20log10s14 dB

A very good Directional Coupler has very large Directivity and Isolation.