ME1000 RF Circuit Design

Tutorial 8

Low Noise Amplifier Design

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

### Question 1

Calculate the mean-square value of shot noise current in a diode with average current of 1mA within a bandwidth of 2MHz.  Draw the equivalent circuit of the diode with the noise source. Take 
q = 1.602  10-19C.

### Question 2

Calculate the mean-square value of thermal noise voltage in a resistor with a resistance of 1k in a bandwidth of 1MHz.  Draw the equivalent circuit of the resistor with the noise source.  Assume room temperature of 300K and k=1.38110-23JK-1.

### Question 3

The wideband RF transistor BFR92A is biased at IC=4.0mA and VCE=10.0V in a CE (common-emitter) configuration.  At 800 MHz, the following small-signal parameters are measured:

Zm=43.352+j31.792 , Fm=1.216 (1.7dB), RI=363.737.

s11 = 0.57<40o

s21 = 3.5<-60o

s12 = 0.1<-30o

s22 = 0.67<-40o

Zo = 50

(a)  Draw the noise figure circles for F=1.8dB and 2.0dB on a Smith Chart.

- Suppose it is required that we need to design a low-noise amplifier with F=2.0dB and power gain of 12 at 800 MHz, find a pair of suitable load and source impedance for this amplifier.

- Determine the overall transducer power gain of the system.