ME1000 RF Circuit Design

Tutorial 9

General Small-Signal

Amplifier Design

(Solution)

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

### Question 1

A silicon bipolar junction transistor amplifier has the following parameters at 800MHz:

Zm=43.352 + j31.792 , Fm=1.216 (0.8493dB), Re=4.737.

S11 = 0.6<148o

S21 = 2.95<-50o

S12 = 0.09<120o

S22 = 0.6<-20o

Zo = 50

Design a low-noise amplifier with power gain not less than 6, noise figure less than 2.0dB and input mismatch better than 2.5.  The load and source impedance connected to the amplifier are 300 and 50 respectively.

Solution

S11 = -0.5088 + 0.318i		S12 = -0.045 + 0.0779i

S21 = 1.8962 - 2.2598i		S22 = 0.5638 - 0.2025i

D = S11∙S22 - S12∙S21

### D = −0.3124+ 0.0342i		|D| = 0.3143

### K = 1−(|S11|)2−(|S22|)2+(|D|)22⋅|S21⋅S12|

K = 0.7134

Since K<1 and |D| <1, the amplifier fails the Rollette Stability Test. It is thus conditionally stable.

Finding the Load Stability Circle parameters

CL:(S22−D⋅S11)(|S22|)2−(|D|)2	RL: S12⋅S21(|S22|)2−(|D|)2

CL:  1.5082 + 1.0993i		RL:  1.0164

Finding the Source Stability Circle parameters

CS:(S11−D⋅S22)(|S11|)2−(|D|)2	RS: S21⋅S12(|S11|)2−(|D|)2

CS:  -1.2467 - 1.3889i		RS:  1.0164

Finding the Constant Power Gain circle parameters

C2 = S22−D⋅S11		C2 = 0.394 – 0.2872i

g2 = 8|S21|2			g2 = 0.9193

Cgp = 1−1−(|S22|)2⋅g2+(|D|)2⋅g2⋅(−g2⋅Re(C2)+i⋅g2⋅Im(C2)

### Cgp =0.292 + 0.2129i

Rgp = 1−2⋅K⋅|S12⋅S21|⋅g2+(|S12⋅S21⋅g2|)2−1−(|S22|)2⋅g2+(|D|)2⋅g2

Rgp = 0.6801

Finding suitable load impedance:

Finding the parameters for noise figure circles

Zsm = 43.352+ i∙31.792		Fm = 1.216		Re_LNA = 4.737

Tsm = Zsm−ZoZsm+Zo = 0.0401 + 0.3269i

For FdB = 2.0

F2dB = 2.0

F1=10F2dB10=1.5849

Ni1=Zo4⋅Re_LNA⋅(F1−Fm)⋅(|1+Tsm|)2=1.1571

Rf1=(Ni1)2+Ni1⋅1−(|Tsm|)21+Ni1		Cf1=Tsm1+Ni1

Cf1 = 0.0186 + 0.1515i		Rf1 = 0.7138

Finding the parameters for input mismatch circle (ZL = 23.67 + j0) and fixing VSWRin = 2.0 < 2.5

ZL = 23.67+ i∙0 VSWRin = 2.0

TL = ZL−ZoZL+Zo=−0.3574

T1 = S11−D⋅TL1−S22⋅TL=−0.5312+0.2424i

M1 = 1−VSWRin−1VSWRin+12=0.8889

Cvswrin = M1⋅T11−(1−M1)⋅(|T1|)2=−0.4908−0.2239i

Rvswrin = 1−M1⋅1−(|T1|)21−(1−M1)⋅(|T1|)2=0.2283

So the final selection for source and load impedance Are:

Zs = 16.90 + j0

ZL = 23.67 + j0

At 800MHz. Actual source and load impedance of

Zs(actual) = 50 + j0 and ZL(actual) = 300 + j0 has to be transformed to the above values using suitable impedance transformation network.

### Question 2

An antenna with a source impedance of 25.845-j100.128 at 4GHz is connected to the input of an amplifier.  The amplifier has the following S-parameters at 4GHz.

Zm=47.5 + j10.8 , Fm=1.2 (0.7918dB), Re=4.0.

S11 = 0.36<148o

S21 = 1.57<27o

S12 = 0.11<42o

S22 = 0.67<-64o

Zo = 50

- Show that the amplifier is unconditionally stable.

- Design a maximum power gain amplifier with the antenna.

- Find the maximum power gain of the system, the transducer gain and the input mismatch factor.

- Also find the noise figure of the system.

- Is the noise figure and mismatch acceptable?  Give your comments.

Solution

S-parameters at 4GHz:

S11 = -0.305 + j0.191

S21 = 1.399 + j0.713

S12 = 0.082 + j0.074

S22 = 0.294 – j0.602

D = S11S22-S21S12 = -0.036 +j0.0786

This shows that the amplifier is unconditionally stable

|D| = 0.087

K=1−|S11|2−|S22|2+|D|22|S21S12|=1.242

Finding the source and load reflection coefficient for maximum power gain:

B1 = S11 – DS22* = -0.247 +j0.190

A1 = 1 + |S11|2 - |S22|2 - |D|2 =0.673

Γsm=12B2[A2−A22−4|B2|2]=0.342+j0.747

Converting the reflection coefficients into impedance:

ZLm = Zo1+ΓLm1−ΓLm=16.382+j75.336

Zsm = Zo1+Γsm1−Γsm=10.899−j16.253

Finding the maximum power gain:

GPmax = |S21||S12|K−K2−1=7.212=8.58dB

GP = GT = GA = GPmax	M =1

To find the noise figure:

Gi=Re|Zm|2=ReRm2+Xm2=0.001686

F=Fmin+GiRsZs−Zm2=1.2+0.00168610.899(45.514)2=1.5204

The noise figure is acceptable, it is considered low (F<1.6 or 2dB is considered low noise figure).