ME1000 RF Circuit Design

Tutorial 10

Large-Signal

Circuits and RF Oscillators

(Solution)

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

Question 3

(a) 	Show that the 3rd order term in the power-series expansion model of a large-signal amplifier

result in signal components that increase with a rate of 3 times the input power level in dBm.

Solution

3rd order term implies the 2nd harmonics.  For single-tone excitation, it is:

14α3A3cos3ωt, where A is the magnitude.

Since the above is a sinusoidal voltage waveform, the average power over a resistance Zo is:

Pave=14α3A322Zo=416α32Zo2A22Zo3

Taking 10log10(Pave/1mW) of the above power, and note that Pin=A22Zo we can show that:

PavddBm=3Pin+Constant

(b) 	The following diagram is captured on a spectrum analyzer of a Power Amplifier (PA) operating at

5 GHz.  The two signal frequency are spaced 1 MHz apart.  Determine the Intermodulation

Distortion and the Third Order Intercept point power levels.  Also estimate the 1dB Gain

Compression power for this PA. NOTE: The vertical scale is 10dB/division.

Solution

Here we will use equation (2.2a) to (2.2c).

The difference between fundamental component and the 3rd order component is around 55 dB, thus IMD = 55 dB.

You can also see from the display of the spectrum analyzer that the marker is on one of the ‘spike’ for 3rd order component, with -60.42 dBm power level (the red color characters).  Thus the fundamental power level, P1 is:

P1 = (-60.42 + 55) dBm = -5.42 dBm.

Using PIP=P1+12IMD, we can find the TOI point PIP:

PIP = (-5.42 + 27.5) dBm = +21.08 dBm.

The 1dB gain compression power is approximately PIP – 10dB = +11.08 dBm.

Question 4

An amplifier has the following power series expansion coefficients at 900 MHz with 50 source and load impedance:

1=12, 2=1.5 and 3=-0.67.  These coefficients are obtained by performing some measurement and employing statistical method.  If one were to use this amplifier to construct a down-conversion mixer, with 900 MHz RF input and 850 MHz LO input, find the conversion gain of this mixer in dB.

Solution

The amplitude of 2nd order component, due to mixing effect is:

α2A1A2

Where A1 is the LO signal amplitude and A2 is the RF signal amplitude.  Let PRF be the average RF power and PIF be average power of the ‘mixed’ component at a resistance Zo.  Then:

PIF=12Zoα2A1A22		PRF=12ZoA22

Thus the conversion gain is:

G=PIFPRF=α2A12

Note: if a number is given for the magnitude of the LO signal, then you should be able to compute it’s value and express the conversion gain in dB.

Question 5

Suppose you have 3 amplifiers A, B and C with properties as shown in the table below.  The amplifiers are cascaded together as shown in Figure Q5.  Determine the maximum output power available from the cascaded amplifiers.  Assume the amplifiers input and output are matched to each other and the system impedance is 50.

| Amplifier Module | Vcc/V | Icc/mA | Small-signal power gain (with 50 source and load impedance) | 1 dB Gain Compression Output Power |
| --- | --- | --- | --- | --- |
| A | 5.0 | 20.0 | 10.0 | +10.0 dBm |
| B | 5.0 | 25.0 | 12.0 | +16.0 dBm |
| C | 5.0 | 30.0 | 10.0 | +24.0 dBm |

A

B

C

A

Cascaded Amplifier 1.

Cascaded Amplifier 2.

Figure Q5.

Solution

For cascaded amplifier 1, the maximum power obtainable from stage A is +10 dBm without excessive harmonic distortion.  This power becomes the input to stage B.  Since stage B has a gain of 12 dB, the theoretical power should be 10 + 12 = 22 dBm.  However since stage B maximum output power is +16 dBm, then the maximum output power will be capped to +16 dBm.  This means the input to stage A has to be limited to 16 – 12 - 10 = -6 dBm (0.4mW into 50), otherwise harmonic distortion will appear at the cascaded amplifier output.

For cascaded amplifier 2, the maximum power obtainable from stage A is +10 dBm without excessive harmonic distortion as usual.  This power becomes the input to stage C.  Since stage C has a gain of 10 dB, the theoretical power should be 10 + 10 = 20 dBm.  This is below stage C maximum output power of +24 dBm, thus the maximum output power will be +22 dBm.  The corresponding input is 20 – 10 – 10 = 0 dBm (1mW into 50).