ME1000 RF Circuit Design

Tutorial 6

Small–Signal

Amplifier Gain and Stability

(Solution)

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

### Question 1

An amplifier is characterized by the following S-parameters at 600MHz, with Zo= 50:

S11 = 0.78<-65o

S21 = 2.2<78o

S12 = 0.11<-21o

S22 = 0.9<-29o

Determine whether the amplifier is unconditionally stable or not.  Also plot the Source and Load Stability Circles on a Smith Chart and identified the stable regions for s and L.

Solution

K = 0.8887

D = -0.1808 – j0.9032, |D| = 0.9212

Amplifier is conditionally stable.

Load Stability Circuit: Radius = 6.28, Center = -5.404-0.280

Source Stability Circuit: Radius = 1.0078, Center = -0.3241+j0.3455

The input side of the amplifier is connected to a voltage source with Vs = 1<0o and source resistance Rs= 65.  The output of the amplifier drives an antenna that has an impedance of ZL= 85+j0 at 600 MHz.  Find the following quantities:

- Power Gains GT, GA, GP at 600 MHz.

Solution

Gp = 16.064

GT = 7.544, GA = 28.728

- Power delivered to the load PL, available power PA and input power to the amplifier Pin and 600 MHz.

Solution

T1 = 0.38093-j0.6481

PA = 1.923mW

PL = GTPA = 14.51mW

Pin = PL/Gp = 0.903mW