ME1000 RF Circuit Design

Tutorial 9

General Small-Signal

Amplifier Design

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

### Question 1

A silicon bipolar junction transistor amplifier has the following parameters at 800MHz:

Zm=43.352 + j31.792 , Fm=1.216 (0.8493dB), Re=4.737.

S11 = 0.6<148o

S21 = 2.95<-50o

S12 = 0.09<120o

S22 = 0.6<-20o

Zo = 50

Design a low-noise amplifier with power gain not less than 6, noise figure less than 2.0dB and input mismatch better than 2.5.  The load and source impedance connected to the amplifier are 300 and 50 respectively.

### Question 2

An antenna with a source impedance of 25.845-j100.128 at 4GHz is connected to the input of an amplifier.  The amplifier has the following S-parameters at 4GHz.

Zm=47.5 + j10.8 , Fm=1.2 (0.7918dB), Re=4.0.

S11 = 0.36<148o

S21 = 1.57<27o

S12 = 0.11<42o

S22 = 0.67<-64o

Zo = 50

- Show that the amplifier is unconditionally stable.

- Design a maximum power gain amplifier with the antenna.

- Find the maximum power gain of the system, the transducer gain and the input mismatch factor.

- Also find the noise figure of the system.

- Is the noise figure and mismatch acceptable?  Give your comments.