ME1000 RF Circuit Design

Lab 2

Power Amplifier Characterization Using Spectrum Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

##### Objective

- To measure the gain, gain compression, harmonic distortion, third-order intercept point, and isolation of a power amplifier (PA) via a spectrum analyzer (SA)

##### Equipment Required

- Keysight N9320B 3 GHz RF Spectrum Analyzer

- 2 × Keysight N9310A RF Signal Generator

- ME1000 RF Transceiver Kit (Transmitter Unit)

##### Accessories Required

- 1 × RF power combiner

- 2 × SMA(m)-to-SMA(m) coaxial cable

- 1 × USB cable

- A PC running Microsoft® Windows XP/Vista®, pre-installed with the RF Trainer Control Panel software

IMPORTANT:

Turn off the training kit when not in use. The kit will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded and the cover is latched up before powering up the device.

# Introduction

## Basic Equipment Setup

Figure 1 – General Equipment Configuration for PA Measurement Using a SA

# Gain, Gain Compression, and Harmonic Distortion Measurements with a Signal Analyzer

Make the connection as shown in Figure 1 to determine the gain, gain compression, and harmonic distortion of the PA. You must use the value of the cable/connector/PCB trace losses from the previous lab.

- Use the following settings for the signal generator (SG):

CW frequency:  		868 MHz

Power:  		–25 dBm

N9310A setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset]

Frequency: 			[Frequency] > [868] > {MHZ}

Amplitude: 			[Amplitude] > [+/-] > [25] > {dBm}

- Use the following settings for the SA:

Centre frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	10 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset/System] > {Preset}

Center frequency: 		[FREQUENCY] > [868] > {MHZ}

Span:	 			[SPAN] > [10] > {MHZ}

Attenuation: 			[AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: 				[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 			[BW/AVG] > {Average}

- On your PC, open the RF Trainer Control Panel. Select the TX Unit from the board selection and click Connect to RF Trainer. In the TX Unit Control area, click Power Amplifier Off/On to power up the PA. You will see the PA PCB LED light up on the trainer, when it is powered on.

#### Exercises

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appear on the SA.

Peak fundamental frequency = __________ MHz

Output power level, Pout_sa_PA = __________ dBm

- What is the measured gain of the PA when the input power is –25 dBm?

Gain @868 MHz   	= Actual PA Output power, Pin – Actual PA Input power, Pout

= (Pout_sa_PA + Loutput_path) − (Psig_gen – Linput_path)

= (__________ dBm + __________ dB) − (−25 dBm − __________ dB)

= __________ dB

Where,

Loutput_path = Lcable_sa + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

Linput_path = Lcable_sg + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

- Compare the results measured with the vector network analyzer (VNA). Are they the same?

- Measure the output power level of the PA at different input power levels.

#### Exercises

- Fill in the table below. Use the cable/PCB/connector losses, L (in dB) found in the previous section.

| SG Input Power , Psig_gen (dBm) | Actual Input to PA, Pin = Psig_gen − Linput_path (dBm) | Output Power Measured at SA, Pout_sa_PA (dBm) | Actual PA Output Power , Pout = Pout_sa_PA + Loutput_path (dBm) |
| --- | --- | --- | --- |
| −15 |  |  |  |
| −14 |  |  |  |
| −13 |  |  |  |
| −12 |  |  |  |
| −11 |  |  |  |
| –10 |  |  |  |
| –9 |  |  |  |
| –8 |  |  |  |
| –7 |  |  |  |
| –6 |  |  |  |
| –5 |  |  |  |
| –4 |  |  |  |
| –3 |  |  |  |
| –2 |  |  |  |
| −1 |  |  |  |
| 0 |  |  |  |

- Plot Pout as a function of Pin in the graph below.

- At what output power level is the gain decreased by 1 dB compared with the gain at a very low input power level?

Output P1dB = __________ dBm

Input P1dB = __________ dBm

- Do the above results correlate with the measurement using a VNA?

- Use the following settings to measure harmonic distortions.

SG settings

CW Frequency:  	868 MHz

Power:  		–20 dBm

N9310A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Frequency: 		[Frequency] > [868] > {MHZ}

Amplitude: 		[Amplitude] > [+/-] > [20] > {dBm}

SA settings

Preset the SA to its default setting.

N9320B setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset/System] > {Preset}

Note: 
This will set the SPAN to Full Span, and the Attenuation and Resolution Bandwidth to Auto.

#### Exercises

- Specify the different frequencies shown on the SA together with their power levels.

Fundamental frequency = __________ MHz, 	Power level = __________ dBm

2nd harmonic frequency = __________ MHz,	Power level = __________ dBm

3rd harmonic frequency = __________ MHz,	Power level = __________ dBm

If the harmonic frequencies are too small to be seen on the SA, set the attenuation to 0 dB.

N9320B setting: [Amplitude] > {Attenuation} > [0] > {dB}

Note: 
To measure the frequency accurately, set the Center Frequency to the frequency of interest, and set the SPAN to 10 MHz or smaller.

- Change the input power level of the SG to the input P1dB measured in the previous section. What will happen to the harmonic frequencies?

# Isolation Measurement

Figure 2 – Equipment Setup for PA Isolation Measurement

Make the connection as shown in Figure 2 by swapping the SG and the SA to determine the isolation of the PA. You must also use the value of the cable/connector/PCB trace losses, Lcable and Lc_pcb from the previous lab. Ensure that the DC supply to the LNA is turned off before attempting to remove any connection. Click Power Amplifier On/Off in the RF Trainer Control Panel to turn off the DC supply to the LNA.

- Use the following settings for the SG:

CW frequency:  		868 MHz

Power:  		–20 dBm

N9310A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Frequency: 		[Frequency] > [868] > {MHZ}

Amplitude: 		[Amplitude] > [+/-] > [20] > {dBm}

- Use the following settings for the SA:

Centre frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	10 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Center frequency: 	[FREQUENCY] > [868] > {MHZ}

Span:	 		[SPAN] > [10] > {MHZ}

Attenuation: 		[AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: 			[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 		[BW/AVG] > {Average}

- In the TX Unit Control area of the RF Trainer Control Panel, click Power Amplifier Off/On to power up the PA. You will see the PA PCB LED light up on the trainer, when it is powered on.

#### Exercises

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency = __________ MHz

Output power level, Pout_sa_PA = __________ dBm

- What is the measured isolation of the PA when input power is –25 dBm?

Isolation @868 MHz   	= Actual PA Output power, Pin – Actual PA Input power, Pout

= (Pout_sa_PA + Losses) − (Psig_gen – Losses)

= (__________ dBm + __________ dB) − (−25 dBm − __________ dB)

= __________ dB

Where, Losses = Lcable + Lc−pcb (cable/connector/PCB trace losses found previously)

- Compare the results measured with the VNA. Are they the same?

# Third Order Intercept Point (TOI or IP3) Measurements

Figure 3 – Two-Tone Measurement Setup for Determining the TOI Point

To measure the TOI point, we need to set up the experiment as shown in Figure 3. When measuring the TOI point, where a large signal is present with a small signal, it is a good practice to set the attenuator as high as possible (up to a level where the noise level is sufficiently below the small signal). When the attenuator is high, the distortion due to the SA is reduced hence ensuring a more accurate measurement.

## Adjusting the Output Power of the RF Power Combiner

Figure 4 – Adjusting the Output Power of Combiner

- Make the connection as shown in Figure 4.

- Use the following settings for the SG separately:

CW frequency 1:  	867.5 MHz

CW frequency 2:  	868.5 MHz

Power level (both):  	About –10 dBm

N9310A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Frequency 1: 		[Frequency] > [867.5] > {MHZ}

Frequency 2: 		[Frequency] > [868.5] > {MHZ}

Amplitude: 		[Amplitude] > [+/-] > [10] > {dBm}

Turn on output: 		[RF On/Off]

- Use the following settings for the SA, but set the internal attenuation of the SA to the maximum:

Centre frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	50 dB

RBW:  			100 kHz

Averaging:  		On

N9320B setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Center frequency: 	[FREQUENCY] > [868] > {MHZ}

Span:	 		[SPAN] > [10] > {MHZ}

Attenuation: 		[AMPLITUDE] > {Attenuation} > [50] > {dB}

RBW: 			[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 		[BW/AVG] > {Average}

- If the signal level of the RF power combiner for both frequencies differs, adjust either one of the SG amplitude setting until you get the same output power (the power output should be about –17.5 dBm if the supplied SMA cables are used) from the RF power combiner for both 867.5 MHz and 
868.5 MHz signals.

- Next, make the connection as shown in Figure 3.

- In the TX Unit Control area of the RF Trainer Control Panel, click Power Amplifier Off/On to power up the PA. You will see the PA PCB LED light up on the trainer when it is powered on.

#### Exercises

- Draw and label the output frequencies and power levels seen on the SA. You may need to adjust the RBW in order to see the intermodulation products.

- Fill in the table below. Start with a 50 dB attenuation and reduce it by 10 dB in steps. Use the extrapolated formula, TOI = (S + losses at output) + D/2 to estimate the TOI point at each attenuation. Losses at output = Lcable + Lc_pcb (cable/connector/PCB trace losses found previously)

| Attenuation | Pout @866.5 MHz | Pout @867.5 MHz | Pout @868.5 MHz | Pout @869.5 MHz | Calculated TOI Point |
| --- | --- | --- | --- | --- | --- |
| 50 dB |  |  |  |  |  |
| 40 dB |  |  |  |  |  |
| 30 dB |  |  |  |  |  |
| 20 dB |  |  |  |  |  |
| 10 dB |  |  |  |  |  |
| 0 dB |  |  |  |  |  |

- Which is the most accurate TOI point for the PA?

# Third Order Intercept Point (TOI or IP3) Verification

Figure 5 – Setup to Verify that the Measured TOI is Due to PA, Not SA

We must now verify that the measured TOI point is due to intermodulation products of the PA, and not due to the intermodulation products of the SA. Remove the PA and connect the output of the combiner directly to the SA as shown in Figure 5.

- Use the following settings for the SG to produce a 1 MHz separation between the input signals:

CW Frequency 1:  	867.5 MHz

CW Frequency 2:  	868.5 MHz

Power level (both):  	Same power level as the previous lab, about –10 dBm

N9310A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Frequency 1: 		[Frequency] > [867.5] > {MHZ}

Frequency 2: 		[Frequency] > [868.5] > {MHZ}

Amplitude: 		[Amplitude] > [+/-] > [10] > {dBm}

Turn on output: 		[RF On/Off]

- Use the following settings for the SA:

Centre frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	50 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Center frequency: 	[FREQUENCY] > [868] > {MHZ}

Span:	 		[SPAN] > [10] > {MHZ}

Attenuation: 		[AMPLITUDE] > {Attenuation} > [50] > {dB}

RBW: 			[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 		[BW/AVG] > {Average}

- Adjust the power level of the two SGs until the output on the SA is the same as Pout @867.5 MHz and Pout @868.5 MHz recorded in the table of the previous test carried out in the TOI point measurement.

- Reduce the attenuation until you see the intermodulation products due to mixer of the SA.

#### Exercises

- At which attenuation does the intermodulation products due to the SA start appearing?

TOI due to SA start to appear when internal attenuator = __________ dB

- Explain why intermodulation products appeared on the SA.

- Which is the more accurate measurement and calculated TOI points when compared to the previous results?

- Increase the attenuation until you see the intermodulation products disappear on the SA. Now, what are the frequencies and power levels display on the SA?

Frequency = __________ MHz, 		Power level = __________ dBm

Frequency = __________ MHz, 		Power level = __________ dBm

- Determine the losses of the cables and RF power combiner.

Losses (L) 	= Input power from SG – Output power level displayed at SA

= Psig_gen1 (dBm) – Pout (dBm)

= __________ dB

Note: 
Take only one of the two frequency tones.

###### References

- Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics,” Keysight Technologies, 2001

- Thomas H. Lee, “Planar Microwave Engineering,” Cambridge University Press, 2004

- David M. Pozar, “Microwave Engineering,” 3rd Edition, John Wiley & Sons, 2005