ME1000 RF Circuit Design

Lab 4

Filter Characterization Using 
Spectrum Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

##### Objective

- To measure insertion loss, bandwidth, and rejection of an RF bandpass filter via a spectrum analyzer (SA)

##### Equipment Required

- Keysight N9320B 3 GHz RF Spectrum Analyzer

- Keysight N9310A RF Signal Generator

- ME1000 RF Transceiver Kit (Transmitter Unit)

##### Accessories Required

- 2 × SMA(m)-to-SMA(m) coaxial cable

IMPORTANT:

Turn off the training kit when not in use. The kit will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded and the cover is latched up before powering up the device.

# Introduction

## Basic Equipment Setup

Figure 1 – General Equipment Configuration for RF Filter Measurement Using a SA

##### Important:

Please use the RF Band-Pass Filter v1.01 on the RF transmitter unit for this lab measurement.

# Insertion Loss Measurement with a SA

Make the connection as shown in Figure 1 to determine the insertion loss of the RF filter. You must use the value of the cable/connector/PCB trace losses, (Lcable and Lc_pcb) from the previous lab.

- Use the following settings for the SG:

CW frequency:  		868 MHz

Power:  		–15 dBm

N9310A setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset]

Frequency: 			[Frequency] > [868] > {MHZ}

Amplitude: 			[Amplitude] > [+/-] > [15] > {dBm}

- Use the following settings for the SA:

Center frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	10 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset/System] > {Preset}

Center frequency: 		[FREQUENCY] > [868] > {MHZ}

Span:	 			[SPAN] > [10] > {MHZ}

Attenuation: 			[AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: 				[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 			[BW/AVG] > {Average}

#### Exercises

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency = __________ MHz

Output power level, Pout_sa_filter = __________ dBm

- What is the measured insertion loss of the filter when the input power is –15 dBm?

Gain @868 MHz 	= Actual Filter Output power, Pout – Actual Filter Input power, Pin

= (Pout_sa_filter + Loutput_path) − (Psig_gen – Linput_path)

= (__________ dBm + __________ dB) − (−15 dBm − __________ dB)

= __________ dB

Where,

Loutput_path = Lcable_sa + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

Linput_path = Lcable_sg + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

Insertion loss @868 MHz = − Gain @868 MHz = __________ dB

- Compare the results measured with the vector network analyzer (VNA). Are they the same?

- Measure the output power level of the filter at different input frequencies.

#### Exercises

- Fill in the table below. Use the cable/PCB/connector losses, L (in dB) found in the previous lab.

| SG Frequency, fsig_gen (MHz) | Actual Input Power Pin = −15 − Linput_path (dBm) | Measured Output Power at SA, Pout_sa_filter (dBm) | Actual Output Power, Pout = Pout_sa_filter – Loutput_path (dBm) | Gain, G = Pout − Pin (dB) |
| --- | --- | --- | --- | --- |
| 750 |  |  |  |  |
| 800 |  |  |  |  |
| 850 |  |  |  |  |
| 900 |  |  |  |  |
| 950 |  |  |  |  |
| 1000 |  |  |  |  |
| 1050 |  |  |  |  |
| 1100 |  |  |  |  |
| 1150 |  |  |  |  |
| 1200 |  |  |  |  |
| 1250 |  |  |  |  |
| 1300 |  |  |  |  |
| 1350 |  |  |  |  |
| 1400 |  |  |  |  |

- Plot the gain, G as a function of the input SG frequency, fsig_gen in the graph below.

- From the previous plot, estimate the 3 dB bandwidth and center frequency of the RF filter.

Estimated lower 3 dB frequency, fup_3db = __________ MHz

Estimated upper 3 dB frequency, flow_3db = __________ MHz

Estimated 3 dB BW = fup_3db − flow_3db = __________ MHz

Center frequency, fc = (fup_3db + flow_3db)/2 = __________ MHz

# Rejection Measurement

- Use the following settings:

SG settings

CW frequency:  		500 MHz

Power:  		–15 dBm

N9310A setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Frequency: 			[Frequency] > [500] > {MHZ}

Amplitude: 			[Amplitude] > [+/-] > [15] > {dBm}

Turn on output: 			[RF On/Off]

SA settings

Center frequency:  	500 MHz

Span:  			10 MHz

Input attenuation:  	10 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Center frequency: 		[FREQUENCY] > [500] > {MHZ}

Span:	 			[SPAN] > [10] > {MHZ}

Attenuation: 			[AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: 				[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 			[BW/AVG] > {Average}

#### Exercises

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency = __________ MHz

Output power level, Pout_sa_filter = __________ dBm

- What is the measured rejection at 500 MHz for the filter when the input power is –15 dBm?

Rejection @500 MHz 	= Actual output power, Pout – Actual input power, Pin

= (Pout_sa_filter + Loutput_path) − (Psig_gen – Linput_path)

= (__________ dBm + __________ dB) − (−15 dBm − __________ dB)

= __________ dB

Where,

Loutput_path = Lcable_sa + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

Linput_path = Lcable_sg + Lhalf_path

(Cable/Connector/PCB trace losses found previously)

- Compare the results measured with the VNA. Are they the same?

- Repeat for a CW frequency of 1736 MHz and 2604 MHz.

Rejection @1736 MHz = __________ dB

Rejection @2604 MHz = __________ dB

###### References

- Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics,” Keysight Technologies, 2001

- Thomas H. Lee, “Planar Microwave Engineering,” Cambridge University Press, 2004

- David M. Pozar, “Microwave Engineering,” 3rd Edition, John Wiley & Sons, 2005