ME1000 RF Circuit Design

Lab 8

Antenna Gain Measurement with
Spectrum Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

##### Objective

- To estimate the gain of an antenna via a spectrum analyzer (SA)

##### Equipment Required

- Keysight N9320B 3 GHz RF Spectrum Analyzer

- Keysight N9310A RF Signal Generator

- 2 × 868 MHz Antenna

# Introduction

## Basic Equipment Setup

Figure 1 – General Equipment Configuration for Antenna Measurement Using a SA

# Gain Estimation with a SA

Make the connection as shown in Figure 1 to estimate the gain of the antenna. Firstly, calculate the far-field distance, Rff. Using 868 MHz:

Wavelength @868 MHz, λ = __________ m (calculated using c/f)

Length of the antenna, D = __________ m

Supplied antenna technical specifications:

Frequency range = 	806 to 960 MHz

Gain = 			2.5 dBi

VSWR =		< 2

Polarization = 		Vertical

Length = 		210 mm

Far-field distance, Rff = __________ m (calculated using 2D2/λ)

- Use the following settings at the signal generator (SG):

CW frequency:  		868 MHz

Power:  		10 dBm

N9310A setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset]

Frequency: 			[Frequency] > [868] > {MHZ}

Amplitude: 			[Amplitude] > [+/-] > [10] > {dBm}

Turn on output:	 		[RF On/Off]

- Use the following settings at the SA:

Centre frequency:  	868 MHz

Span:  			10 MHz

Input attenuation:  	10 dB (or auto)

RBW:  			100 kHz (or auto)

Averaging:  		On

N9320B setting: 		“[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: 	[Preset/System] > {Preset}

Center frequency: 		[FREQUENCY] > [868] > {MHZ}

Span: 				[SPAN] > [10] > {MHZ}

Attenuation: 			[AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: 				[BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: 			[BW/AVG] > {Average}

- Make sure that the distance between the transmitter and receiver antennas, R is slightly more than the far-field distance, Rff.

#### Exercises

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency = __________ MHz

Output power level, Pr = __________ dBm

- Calculate the gain of the antenna using Friss equation as shown below:

Pr (dBm)= Gr (dB)+ Gt (dB)+ Pt (dBm) -10log4πRλ2

Gr (dB)+ Gt (dB)=Pr (dBm) - Pt (dBm)+10log4πRλ2

2G (dB)=Pr (dBm) - Pt (dBm)+10log4πRλ2 , where Gr = Gt = G

= __________ dBm − 10 dBm + __________ dB

Antenna Gain, G = __________ dB

- Repeat the calculation at different frequencies and distance, R.

#### Exercises

- Fill in the table below.

| SG Frequency, (MHz) | Wavelength  = c/f (m) | Calculated Far-Field Distance, Rff (m) | Distance Between Antennas, R (m) | Measured Output Power at SA, Pr (dBm) | Calculated Gain, G (dB) |
| --- | --- | --- | --- | --- | --- |
| 750 |  |  | 1.5 × Rff = |  |  |
| 750 |  |  | 2 × Rff = |  |  |
| 750 |  |  | 2.5 × Rff = |  |  |
| 868 |  |  | 2 × Rff = |  |  |
| 868 |  |  | 2.5 × Rff = |  |  |
| 1100 |  |  | 1.5 × Rff = |  |  |
| 1100 |  |  | 2 × Rff = |  |  |
| 1100 |  |  | 2.5 × Rff = |  |  |
| 1200 |  |  | 1.5 × Rff = |  |  |
| 1200 |  |  | 2 × Rff = |  |  |
| 1200 |  |  | 2.5 × Rff = |  |  |
| 1300 |  |  | 1.5 × Rff = |  |  |
| 1300 |  |  | 2 × Rff = |  |  |
| 1300 |  |  | 2.5 × Rff = |  |  |

- Does the calculated gain remain approximately the same when the distance between the antennas is different? Explain your answer.

##### Assumptions in this lab:

- Both the transmitter and receiver antennas are identical and have the same gain.

- Reflection is not taken into consideration when calculating the antenna gain.

###### References

- David M. Pozar, “Microwave Engineering,” 3rd Edition, John Wiley & Sons, 2005