M1000 RF Circuit Design

Lab 2

Power Amplifier Characterization Using
Vector Network Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

##### Objectives

- To characterize the power amplifier (PA) with reflection and transmission measurements via a vector network analyzer (VNA)

- To measure the gain compression using power sweeps via a VNA

##### Equipment Required

- Keysight E5061A ENA-L RF Network Analyzer

- ME1000 RF Transceiver Kit (Transmitter Unit)

##### Accessories Required

- 2 × SMA(m)-to-SMA(m) coaxial cable

- 1 × USB cable

- A PC running Microsoft® Windows XP/Vista®, pre-installed with the RF Trainer Control Panel software

IMPORTANT:

Turn off the training kit when not in use. The kit will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded and the cover is latched up before powering up the device.

# Introduction

## Basic Equipment Setup

Figure 1 – General Equipment Configuration for PA Measurement Using a VNA

##### Important:

In measuring a high gain amplifier, it is possible to overload the amplifier input. This will cause the unit to saturate and cause a significant non-linearity in the output signal, which will possibly damage the unit. It can also damage the test equipment due to the high power amplification. You should either add more attenuation to the input or output of the amplifier or reduce the RF power of the signal source before repeating the measurement.

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall> on the VNA. Using the knob, select the saved calibration data in the nonvolatile memory and press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, use the <Scale>, <Format>, and <Marker> menus to help you navigate with ease. Use the AUTO SCALE option under the <Scale> menu, to help you manipulate the display.

Note that if you press <Preset>, you will need to recall your calibration. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM through PCB board for transmission. Similarly, the above calibration can be performed using appropriate calibration standards. (Preset also sets the “Calibration Kit” selection to the default kit that generally does not correspond to the TRM PCB board you will use.)

##### Note:

Use the marker to record down your readings. Marker(s) is/are used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for maximum and minimum points.

# Transmission Measurements

- Recall the calibration data (saved in the previous lab) from the nonvolatile memory.

- Connect the PA as shown in Figure 1.

- Set the following settings to reduce the input source power to the PA:

Power:		Source power, −40dBm

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Source power: 		[Sweep Setup] > {Power} > {Power} > [± 40] > [x1]

- On your PC, launch the RF Trainer Control Panel. Select the TX Unit from the board selection and click Connect to RF Trainer. In the TX Unit Control area, click Power Amplifier Off/On to power up the PA. You will see the PA PCB LED light up on the trainer kit when it is powered on.

- Use the following settings to determine the small signal gain of the PA for a frequency range of between 700 MHz and 1 GHz:

Start:  		700 MHz

Stop:  		1 GHz

Format:		Log magnitude

Measure:  	S21

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Start frequency: 	[Start] > [700] > [M/μ]

Stop frequency: 	[Stop] > [1] > [G/n]

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S21}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Find the maximum gain and the frequency at which it occurs.

Gmax = __________ dB at __________ MHz

- Find the frequency below this at which the gain is reduced by 3 dB from the maximum gain.

Gmax – 3 dB occurs at __________ MHz

- Find the gain at 868 MHz.

Gain @868 MHz = __________ dB

- Change the following settings to determine the isolation of PA:

Format:  	Log magnitude

Measure:  	S12

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S12}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Find the isolation at 868 MHz.

Isolation @868 MHz = __________ dB

Note: 
Use averaging, smoothing, and smoothing aperture if necessary.

- Elaborate the importance of isolation in an amplifier.

# Reflection Measurements

- Use the following settings to determine the input return loss of the PA:

Format:  	Log magnitude

Measure:  	S11

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S11}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Find the minimum input return loss and the frequency at which it occurs.

Minimum return loss = __________ dB at __________ MHz

- Find the maximum input return loss and the frequency at which it occurs.

Maximum return loss = __________ dB at __________ MHz

- Find the input return loss at 868 MHz.

Return loss @868 MHz = __________dB

Note: 
Use the marker function to search for the maximum and minimum values.

- Use the following settings to determine the output return loss of the PA:

Format:  	Log magnitude

Measure:  	S22

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S22}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Find the minimum output return loss and the frequency at which it occurs.

Minimum return loss = __________ dB at __________ MHz

- Find the maximum output return loss and the frequency at which it occurs.

Maximum return loss = __________ dB at __________ MHz

- Find the output return loss at 868 MHz.

Return loss @868 MHz = __________ dB

Note: 
Use the marker function to search for the maximum and minimum values.

# Gain Compression Measurement with Power Sweep

1.	Perform a power sweep at 868 MHz using the following settings:

Sweep setup:  		CW Frequency to 868 MHz

Sweep Type to Power Sweep

Power:  		Power Range (–15 dBm to 0 dBm)

Start:  			–15 dBm

Stop:  			0 dBm

Measure:  		S21

Scale: 			Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Sweep type: 		[Sweep Setup] > {Sweep Type} > {Power Sweep}

CW frequency: 		{Power} > {CW Frequency} > [868] > [M/μ]

Power ranges: 		{Power Ranges} > {-15 to 0}

Measure: 		[Meas] > {S21}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Define and elaborate the meaning of the 1 dB compression point.

- Plot the gain vs. power sweep graph. Use a marker to locate the P1 dB point.

- What is the VNA setting at the 1 dB compression point?

P1 dB = __________ dBm

- What is the measured gain at 1 dB compression?

Gain @P1 dB = __________ dB

- The P1 dB point measured above refers to the input or output point?

###### References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis,” Keysight Technologies

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers,” Keysight Technologies

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements,” Keysight Technologies

- Application Note 1287-4, “Network Analyzer Measurements: Filter and Amplifier Examples,” Keysight Technologies

- Product Note 8753-1, “RF Component Measurements: Amplifier Measurements Using the Keysight 8753 Network Analyzer,” Keysight Technologies

- Thomas H. Lee, “Planar Microwave Engineering,” Cambridge University Press, 2004