ME1000 RF Circuit Design

Lab 6

Antenna Reflection Measurement with
Vector Network Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

##### Objective

- To characterize the antenna with reflection measurement via a vector network analyzer (VNA)

##### Equipment Required

- Keysight E5061A ENA-L RF Network Analyzer

- 2 × 868 MHz Antenna

# Introduction

## Basic Equipment Setup

Figure 1 – Antenna-Under-Measurement

##### Important:

There are multiple measurement channels on the VNA, which can be thought of as having multiple VNAs in a single equipment. You might want to use Channel 1 for transmission measurements and Channel 2 for reflection measurements.

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall> on the VNA. Using the knob, select the saved calibration data in the nonvolatile memory and press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, use the <Scale>, <Format>, and <Marker> menus to help you navigate with ease. Use the AUTO SCALE option under the <Scale> menu, to help you manipulate the display.

Note that if you press <Preset>, you will need to recall your calibration. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM through PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “Calibration Kit” selection to the default kit that generally does not correspond to the TRM PCB board you will use.)

##### Note:

Use the marker to record down your readings. Marker(s) is/are used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for maximum and minimum points.

# Reflection Measurements

- Recall the calibration data (saved in the previous lab) from the nonvolatile memory. If this setup is different from the previous one or no previous calibration was done, please perform reflection calibration (depending on whether S11 or S22 is used). It is recommended to connect the antenna directly to the VNA, as this setup is able to hold the antenna firmly at its fixed position.

##### For reflection calibration:

Press Cal, Calibrate, then S11 or S22. Connect the provided TRM short PCB board to the VNA port and then press Short to start the calibration. Alternatively, appropriate cal standards can be used to perform the required calibration.

- Connect the antenna flat as shown in Figure 1(a).

- Use the following settings to determine the return loss of the antenna:

Format:  	Log magnitude

Measure:  	S11

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S11}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Plot the minimum return loss, S11 vs. the frequency. Use the marker function to determine the minimum point.

Minimum return loss, S11 = __________ dB at frequency = __________ MHz

- From the minimum return loss observed, describe the frequency (or frequencies) tuned by the antenna.

- Using the equation c = fλ, estimate the required length of the wired antenna on the tuned frequency. Does it correlate with the measured return loss? Explain your answer.

- Change the VNA setting to display the SWR.

Format:  	SWR

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {SWR}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- What is the standing wave ratio at the tuned frequency?

SWR @tuned frequency = 1:__________

- Change the VNA setting to display the Smith Chart plot for impedance measurement.

Format:  	Smith Chart

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Smith} > {R+jX}

#### Exercises

- What is the impedance at the tuned frequency?

Impedance at tuned frequency = (__________+ j __________) Ω

- What is the ideal impedance at the tuned frequency?

Ideal impedance at passband = (__________ + j __________) Ω

- Bend the antenna to a vertical position as shown in Figure 1(b). Change the VNA settings to display the return loss measurement.

Format:  	Log magnitude

Measure:  	S11

Scale:  		Auto scale

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Log Mag}

Measure: 		[Meas] > {S11}

Scale: 			[Scale] > {Auto Scale}

#### Exercises

- Plot the minimum return loss, S11 vs. the frequency. Use the marker function to determine the minimum point.

Minimum return loss, S11 = __________ dB at frequency = __________ MHz

- From the minimum return loss observed, describe the frequency (or frequencies) tuned by the antenna.

- Are the results similar with the previous measurement where the antenna is in a flat position? Comment and explain your answer.

- Change the VNA setting to display the SWR.

Format:  	SWR

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {SWR}

Scale: 			[Scale] > {Auto Scale}

#### Exercise

- What is the standing wave ratio at the tuned frequency?

SWR @tuned frequency = 1:__________

- Change the VNA setting to display the Smith Chart for impedance measurement.

Format:  	Smith Chart

E5061A setting: 	“[ ]”: Hardkey; “{ }”: Softkey

Format: 		[Format] > {Smith} > {R+jX}

#### Exercises

- What is the impedance at the tuned frequency?

Impedance at tuned frequency = (__________ + j __________) Ω

- What is the ideal impedance at the tuned frequency?

Ideal Impedance at passband = (__________ + j __________) Ω

###### References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis,” Keysight Technologies

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers,” Keysight Technologies

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements,” Keysight Technologies