ME1000 RF Circuit Design

Lab 5

Low Noise Amplifier (LNA) Characterization

using Spectrum Analyzer (SA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

i.	To measure gain and isolation of a LNA via a SA.

Equipments and Accessories Required:

- Spectrum analyzer

- Signal generator

- ME1000 Receiver unit

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 5 – LNA Characterization using SA

Basic Equipment Setup

Required accessories

SMA m-m coaxial cable

USB cable

PC/Notebook with RF Trainer Control Panel installed

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

Spectrum Analyzer

RF Input

CW Signal Generator

RF Output

SMA cable

Psig_gen

Pin

Pout

SMA cable

Pout_sa_LNA

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Figure 1 General equipment configuration for LNA measurement using an SA

Gain measurement with SA

Make the connection as shown in Figure 1 to determine the gain of the LNA. You must use the value of the cable/connector/PCB trace losses from the previous lab.

- Use the following settings for the signal generator (SG):

CW Frequency 	:  868 MHz

Power		 	:  –15 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset]

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [15] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for the SA:

Centre Frequency 	:  868 MHz

Span 		 	:  10 MHz

Input Attenuation 	:  10 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset/System] > {Preset}

## Center Freq: [FREQUENCY] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

- On PC/Notebook, launch the RF Trainer Control Panel. Select RX Unit from the board selection and click “Connect to RF Trainer”. In the RX Unit Control area, click “LNA Off” to power up the LNA. You will see the LNA PCB LED light up on the trainer, and the button text change to “LNA On”.

OBSERVATIONS/DATA RECORDING

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency          = 868 MHz

Output power level, Pout_sa_LNA  = −7.15 dBm

- What is the measured gain of the LNA when input power is –15dBm?

Gain @868MHz   = Actual LNA Output power, Pout – Actual LNA Input power, Pin

= (Pout_sa_LNA + Loutput_path) − (Psig_gen – Linput_path)

= (−7.15 dBm + 1.185 dB) − (−15 dBm − 1.165 dB)   = 10.2 dB

where,

Loutput_path = Lcable_sa + Lhalf_path (cable/connector/PCB trace losses found previously)

Linput_path = Lcable_sg + Lhalf_path (cable/connector/PCB trace losses found previously)

- Compare the results measured with vector network analyzer (VNA). Are they the same?

In theory, they should be the same. However, in practical environment, the results will not be the same. VNA normally provide better accuracy due to its vector correction/calibration.

Isolation measurement

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

Spectrum Analyzer

RF Input

CW Signal Generator

RF Output

SMA cable

Psig_gen

Pin

Pout

SMA cable

Pout_sa_LNA

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Figure 2 Setup for LNA isolation measurement

Make the connection as shown in Figure 2 by swapping the SG and SA, in order to determine the isolation of the LNA. You must also use the value of cable/connector/PCB trace losses, (Lcable and Lc_pcb) from the previous lab. Ensure that the DC supply to the LNA is turned off before attempting to remove any connection. Click “LNA On” in the RF Trainer Control Panel to turn off the DC supply to the LNA.

- Use the following settings for the SG:

CW Frequency 	:  868 MHz

Power		 	:  –15 dBm

###### N9310A setting: “[ ]” : Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [15] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for the SA:

Centre Frequency 	:  868 MHz

Span 		 	:  10 MHz

Input Attenuation 	:  10 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Freq: [FREQUENCY] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

- On PC/Notebook, launch the RF Trainer Control Panel. Select RX Unit from the board selection and click “Connect to RF Trainer”. In the RX Unit Control area, click “LNA Off” to power up the LNA. You will see the LNA PCB LED light up on the trainer, and the button text change to “LNA On”.

OBSERVATIONS/DATA RECORDING

- What is the frequency and power display on the spectrum analyzer? Ignore the harmonic frequencies that appeared on the spectrum analyzer.

Fundamental frequency   = 868 MHz

Output power level           = −33.15 dBm

- What is the measured isolation of the LNA when input power is –15dBm?

Isolation @868 MHz = Actual LNA Output power, Pout – Actual LNA Input power, Pin

= (Pout_sa_LNA + Loutput_path) − (Psig_gen – Linput_path)

= (−33.15 dBm + 1.185 dB) − (−15dBm − 1.165 dB)   = −15.8 dB

In this case Pout should be < Pin, isolation should be at least -10 dB. Ideally should be < −16dB for this LNA.

- Compare the results measured with VNA. Are they the same?

Yes, it should be almost the same but can differ due to better calibration in VNA.

References

- Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics”, Keysight Technologies, 2001

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.

- David M. Pozar, “Microwave Engineering”, 3rd Edition, John Wiley & Sons, 2005.

- Application Note 1354, “Practical Noise-Figure Measurement and Analysis for Low-Noise Amplifier Designs”, Keysight Technologies.