ME1000 RF Circuit Design

Lab 3

Power Amplifier (PA) Characterization

using Spectrum Analyzer (SA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

- To measure gain, gain compression, harmonic distortion, third order intercept point, and isolation of PA via a SA.

Equipments and Accessories Required:

- Spectrum analyzer

- Signal generator

- ME1000 Transmitter unit

- Power combiner

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 3 – PA Characterization using SA

Basic Equipment Setup

Required accessories

Spectrum Analyzer

RF Input

CW Signal Generator

RF Output

PC/

Notebook

RF Trainer Control Panel

SMA cable

SMA cable

USB cable

Psig_gen

Pout_sa_PA

RF band-pass filter

Power

amplifier

Up converter

Frequency synthesizer

RF Out

LO In

RF In

IF Out

In

Out

Out

In

USB port

Transmitter unit

Pin

Pout

USB cable

SMA m-m coaxial cable

PC/Notebook with RF Trainer Control Panel installed

Power combiner

(* for TOI)

Figure 1 General equipment configuration for PA measurement using an SA

Gain, gain compression and harmonic distortion measurements with an SA

Make the connection as shown in Figure 1 to determine the gain, gain compression and harmonic distortion of the PA. You must use the value of cable/connector/PCB trace losses from the previous lab.

- Use the following settings for the signal generator (SG):

CW Frequency 	:  868 MHz

Power		 	:  –25 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset]

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [25] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for the SA:

Centre Frequency 	:  868 MHz

Span 		 	:  10 MHz

Input Attenuation 	:  10 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset/System] > {Preset}

## Center Frequency: [Frequency] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [10] > {dB}

RBW: [BW/Avg] > {Res BW} > {100} > {KHZ}

Averaging: [BW/Avg] > {Average}

- On PC/Notebook, open the RF Trainer Control Panel. Select TX Unit from the board selection and click “Connect to RF Trainer”. In the TX Unit Control area, click “Power Amplifier Off” to power up the PA. You will see the PA PCB LED light up on the trainer, and the button text change to “Power Amplifier On”.

OBSERVATIONS/DATA RECORDING

- What is the peak frequency and power display on the SA? Ignore those harmonic frequencies that appear on the SA.

Peak fundamental frequency = 868 MHz

Output power level, Pout_sa_PA = −11.8 dBm

- What is the measured gain of the PA when input power is –25dBm?

Gain @868 MHz   = Actual PA Output power, Pin – Actual PA Input power, Pout

= (Pout_sa_PA + Loutput_path) − (Psig_gen – Linput_path)

= (−11.8 dBm + 1.185 dB) − (−25 dBm − 1.165 dB)   = 15.5 dB

where,

Loutput_path = Lcable_sa + Lhalf_path (cable/connector/PCB trace losses found previously)

Linput_path = Lcable_sg + Lhalf_path (cable/connector/PCB trace losses found previously)

- Compare the results measured with VNA. Are they the same?

In theory, they should be the same. However, in practical environment, the results will not be the same. VNA usually provides better accuracy due to its vector correction/calibration.

- Measure the output power level of the PA at different input power levels.

OBSERVATIONS/DATA RECORDING

- Fill in the table below. Use cable/PCB/connector losses, L (in dB) found in previous lab.

| SG Input power , Psig_gen (dBm) | Actual input to PA, Pin = Psig_gen − Linput_path (dBm) | Output power measured at SA, Pout_sa_PA (dBm) | Actual PA output power, Pout = Pout_sa_PA + Loutput_path (dBm) |
| --- | --- | --- | --- |
| −15 | −16.185 | 0.15 | 1.315 |
| −14 | −15.185 | 1.07 | 1.235 |
| −13 | −14.185 | 2.04 | 3.205 |
| −12 | −13.185 | 3.02 | 4.185 |
| −11 | −12.185 | 3.97 | 5.135 |
| –10 | −11.185 | 4.92 | 6.085 |
| –9 | −10.185 | 5.85 | 7.015 |
| –8 | −9.185 | 6.76 | 7.925 |
| –7 | −8.185 | 7.60 | 8.765 |
| –6 | −7.185 | 8.38 | 9.545 |
| –5 | −6.185 | 9.04 | 10.205 |
| –4 | −5.185 | 9.58 | 10.745 |
| –3 | −4.185 | 10.01 | 11.175 |
| –2 | −3.185 | 10.29 | 11.455 |
| −1 | −2.185 | 10.37 | 11.535 |
| 0 | −1.185 | 10.40 | 11.565 |

- Plot Pout as a function of Pin in the graph below.

c)	At what output power level is the gain decreased by 1 dB compared with the gain at a very low input power level?

Output P1dB   = 11.2 dBm

Input P1dB     = −5.3 dBm

- Do the above results correlate with the measurement taken using a VNA?

Yes, it should be almost the same but can differ due to better calibration in VNA

- Use the following settings to measure harmonic distortions.

SG settings:

CW Frequency 	:  868 MHz

Power		 	:  –20 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [20] > {dBm}

Turn on output: [RF On/Off]

SA settings:

Preset the SA to its default setting

N9320A/B setting: “[ ]”: Front Panel key; “{ }”: Softkey

Preset: [Preset/System] > {Preset}

This will set the SPAN to Full Span, Attenuation and Resolution Bandwidth to Auto

OBSERVATIONS/DATA RECORDING

- Specify the different Frequencies shown on the SA together with their power levels.

Fundamental frequency = 868 MHz, 	Power level = −5.60 dBm

2nd harmonic frequency = 1736 MHz,	Power level = −60.0 dBm

3rd harmonic frequency = 2604 MHz,	Power level = −65.0 dBm

If the harmonic frequencies are too small to be seen on the SA, set the attenuation to 0 dB.

N9320A setting: [Amplitude] > {Attenuation} > [0] > {dB}

Note:

To measure frequency accurately, set the Center Frequency to the frequency of interest and set the SPAN to 10MHz or smaller.

- Change the input power level of the signal generator to the input P1dB measured in the previous section. What will happen to the harmonic frequencies?

At P1dB, the amplifier will start to saturate, therefore the power level of the harmonic frequencies will increase higher or steeper compare to before P1dB.

Isolation measurement

Spectrum Analyzer

RF Input

CW Signal Generator

RF Output

PC/

Notebook

RF Trainer Control Panel

SMA cable

SMA cable

USB cable

Psig_gen

Pout_sa_PA

RF band-pass filter

Power

amplifier

Up converter

Frequency synthesizer

RF Out

LO In

RF In

IF Out

In

Out

Out

In

USB port

Transmitter unit

Pin

Pout

Figure 2 Setup for PA isolation measurement

Make the connection as shown in Figure 2 by swapping the SG and SA, in order to determine the isolation of the PA. You must also use the value of cable/connector/PCB trace losses, (Lcable and Lc−pcb) from the previous lab. Ensure that the DC supply to the LNA is turned off before attempting to remove any connection. Click “Power Amplifier On” in the RF Trainer Control Panel to turn off the DC supply to the LNA.

- Use the following settings for SG:

CW Frequency 	:  868 MHz

Power		 	:  –20 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [20] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for SA:

Centre Frequency 	:  868 MHz

Span 		 	:  10 MHz

Input Attenuation 	:  10 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Frequency: [FREQUENCY] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} >[10] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

- In the TX Unit Control area of the RF Trainer Control Panel, click “Power Amplifier Off” to power up the PA. You will see the PA PCB LED light up on the trainer, and the button text change to “PA On”.

OBSERVATIONS/DATA RECORDING

- What is the peak frequency and power display on the SA? Ignore the harmonic frequencies that appeared on the SA.

Peak fundamental frequency = 868 MHz

Output power level, Pout_sa_PA = −48.12 dBm

- What is the measured isolation of the PA when input power is –25dBm?

Isolation @868 MHz   = Actual PA Output power, Pin – Actual PA Input power, Pout

= (Pout_sa_PA + Losses) − (Psig_gen – Losses)

= (−48.2 dBm + 1.185 dB) − (−25 dBm − 1.165 dB) = −21.05 dB

where Losses = Lcable + Lc−pcb (cable/connector/PCB trace losses found previously)

c)	Compare the results measured with the VNA. Are they the same?

Yes, it should be almost the same but can differ due to the better calibration in VNA.

Third Order Intercept Point (TOI or IP3) Measurements

Spectrum Analyzer

RF Input

PC/

Notebook

RF Trainer Control Panel

SMA cable

SMA cable

USB cable

Psig_gen

Pout_sa_PA

RF band-pass filter

Power

amplifier

Up converter

Frequency synthesizer

RF Out

LO In

RF In

IF Out

In

Out

Out

In

USB port

Transmitter unit

Pout

CW Signal Generator 2

RF Output

Power combiner

CW Signal Generator 1

RF Output

Figure 3 Two tone measurement setup for determining TOI

To measure TOI, we need to setup the experiment as shown in Figure 3. When measuring TOI where large signal is present with small signal, it is a good practice to set the attenuator as high as possible (up to the level where noise level is sufficiently below the small signal). When the attenuator is high, the distortion due to SA is reduced hence ensuring a more accurate measurement.

#### Adjusting the output power of combiner

Spectrum Analyzer

RF Input

SMA cable

CW Signal Generator 2

RF Output

Power combiner

CW Signal Generator 1

RF Output

Figure 4 Adjusting the output power of combiner

- Make the connection as shown in Figure 4.

- Use the following settings for SG separately:

CW Frequency 1 	:  867.5 MHz

CW Frequency 2 	:  868.5 MHz

Power level (both)	:  About –10 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency1: [Frequency] > [867.5] > {MHZ}

Frequency2: [Frequency] > [868.5] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [10] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for SA but the internal attenuation of the SA is set to maximum:

Centre Frequency 	:  868 MHz

Span 		 	:  10MHz

Input Attenuation 	:  50 dB

RBW 			:  100 kHz

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Frequency: [FREQUENCY] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [50] > {dB}

RBW: [BW/Avg] > {Res BW} > {100} > {KHZ}

Averaging: [BW/Avg] > {Average}

4.	If the signal level of the power combiner for both frequencies differs, adjust either one of the signal generator amplitude setting till you get the same output power (the power output should be about –17.5dBm if the supplied SMA cables are used) from the combiner for both 867.5MHz and 868.5MHz signals.

5.	Next, make the connection as shown in Figure 3.

- In the TX Unit Control area of the RF Trainer Control Panel, click “Power Amplifier Off” to power up the PA. You will see the PA PCB LED light up on the trainer, and the button text change to “Power Amplifier On”.

RF Trainer Control Panel

OBSERVATIONS/DATA RECORDING

- Draw and label the output frequencies and power levels seen on the SA. You may need to adjust the RBW in order to see the intermodulation products.

Frequency (MHz)

868 MHz

S

D

- Fill in the table below. Start with 50dB attenuation and reduce by 10dB in steps. Use the extrapolated formula, TOI = (S + losses at output) + D/2 to estimate the TOI at each attenuation. Losses at output = Lcable + Lc−pcb (cable/connector/PCB trace losses found previously)

| Attenuation | Pout @866.5 MHz | Pout @867.5 MHz | Pout @868.5 MHz | Pout @869.5 MHz | Calculated TOI |
| --- | --- | --- | --- | --- | --- |
| 50 dB | −47.80 | −2.79 | −2.79 | −48.60 | 22.27 |
| 40 dB | −47.64 | −2.85 | −2.86 | −48.32 | 22.05 |
| 30 dB | −47.68 | −2.99 | −3.01 | −48.16 | 21.82 |
| 20 dB | −47.78 | −3.02 | −3.04 | −48.34 | 21.83 |
| 10 dB | −47.56 | −2.97 | −2.96 | −48.04 | 21.80 |
| 0 dB | −43.12 | −2.68 | −2.67 | −43.62 | 20.02 |

c) Which is the most accurate TOI point for the PA?

The accuracy depends on the saturation of the internal mixer in SA. If input power is too high, it may cause the mixer to go into saturation. If so, the measured TOI will not be accurate as non-linear responses will be generated at the output of the mixer. The most accurate TOI will be when higher attenuation is used.

Third Order Intercept Point (TOI or IP3) verification

Spectrum Analyzer

CW Signal Generator 1

RF Output

RF Input

Psig_gen_1

Power Combiner

Pout

Psig_gen_2

CW Signal Generator 2

RF Output

Figure 5 Setup to verify that the measured TOI is due to PA, not SA

We must now verify that the measured TOI is due to intermodulation products of the PA, and not due to the intermodulation products of SA. Remove the PA and connect the output of the combiner directly to the SA as shown in Figure 5.

- Use the following settings for SG to produce a 1 MHz separation between input signals:

CW Frequency 1 	:  867.5 MHz

CW Frequency 2 	:  868.5 MHz

Power level (both)	:  Same power level as the previous lab, about –10dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }” :Softkey

Frequency1: [Frequency] > [867.5] > {MHZ}

Frequency2: [Frequency] > [868.5] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [10] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for SA:

Centre Frequency 	:  868 MHz

Span 		 	:  10MHz

Input Attenuation 	:  50 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Frequency: [Frequency] > [868] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [50] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

- Adjust the power level of the two SGs until the output on SA is the same as Pout @867.5 MHz and Pout @868.5 MHz recorded in the table of the previous test carried out in the TOI measurement.

- Reduce the attenuation until you see the intermodulation products due to mixer of the SA.

OBSERVATIONS/DATA RECORDING

a)   At which attenuation the intermodulation products due to SA start appearing?

TOI due to SA start to appear when internal attenuator = 10 dB

The intermodulation signals become more apparent at lower attenuation level of 5 dB and below. It depends on the SA used and the input power setting.

- Explain why intermodulation products appeared on the SA.

Non-linearity of the mixer appears to be significant if the input power into the mixer is too high. This causes the mixer goes into saturations. Attenuator is usually used to prevent this from happening.

- Now, which is the most accurate measured and calculated TOI based on previous results?

The most accurate will be when the highest attenuation is used in SA during measurement.

- Increase the attenuation until you see the intermodulation products disappear on the SA. Now, what are the frequencies and power levels display on the SA.

With the following setting:

SG1:	Frequency = 867.5 MHz, Power level = 4 dBm

SG2: 	Frequency = 868.5 MHz, Power level = 5.3 dBm,

the SA displays two power levels at −2.9dBm at 867.5MHz and 868.5MHz when the input attenuation is 20dB.

- Determine the losses of the cables and power combiner.

Losses = Input power from signal generator – Output power level displayed on SA

L = Psig_gen1 (dBm) – Pout (dBm) = 4 − (−2.8) = 6.8 dB for 867.5 MHz

L = Psig_gen1 (dBm) – Pout (dBm) = 5.3 − (−2.8) = 8.1 dB for 868.5 MHz

[Note: You can take one of the frequency of the two tones.]

References

- Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics”, Keysight Technologies, 2001

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.

- David M. Pozar, “Microwave Engineering”, 3rd Edition, John Wiley & Sons, 2005.