ME1000 RF Circuit Design

Lab 12

Frequency Synthesizer Characterization

using Spectrum Analyzer (SA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

i.	To measure output power level, harmonic frequencies, and phase noise of a frequency synthesizer.

Equipments and Accessories Required:

- Spectrum analyzer

- ME1000 Receiver unit

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 12 – Frequency Synthesizer Characterization using SA

Basic Equipment Setup

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

Spectrum Analyzer

RF Input

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Plo

Psa_LO

Required accessories

SMA m-m coaxial cable

USB cable

PC/Notebook with RF Trainer Control Panel installed

Figure 1 General equipment configuration for frequency synthesizer measurement using a SA

Determine the local oscillator (LO) input level using a SA

- Connect the frequency synthesizer directly to the SA, as shown in Figure 1.

- On PC/Notebook, launch the RF Trainer Control Panel. Select RX Unit from the board selection and click “Connect to RF Trainer”. In the Frequency Synthesizer Control area, click “Frequency Synthesizer Off” to power up the synthesizer. You will see the frequency synthesizer PCB LED light up on the trainer, and the button text change to “Frequency Synthesizer On”. Set the frequency to 818MHz.

- Set the following setting for the SA:

Centre Frequency	:  818 MHz

Span			:  10 MHz

RBW			:  100 kHz

Attenuation		:  40 dB

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset/System] > {Preset}

## Center Freq: [FREQUENCY] > [818] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [40] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

OBSERVATIONS/DATA RECORDING

- What is the power level displayed on the SA?

LO level measured at SA, Psa_LO @818 MHz = −5.2 dBm

[Use marker function – search peak if necessary]

- Determine the actual output power level of the LO.

Actual LO output power, Plo = Psa_LO + Lcable

= −5.2 dBm + 1.025 dB = −4.175 dBm

- Change Span to Full Span in order to look at the harmonic tones.

N9320A/B setting: [SPAN] > {Full Span}

## Attenuation: [AMPLITUDE] > {Attenuation} > [10] or {auto} > {dB}

OBSERVATIONS/DATA RECORDING

- Sketch the harmonic tones and measure their respective power level.

The observed signals are −5.2 dBm at 818 MHz, −21.8 dBm at 1.64 GHz and −28.02 dBm at 2.46 GHz.

Phase noise measurement

- Use the same connection as shown in Figure 1.

- Use the following setting for SA:

Centre Frequency 	:  818 MHz

Span		 	:  300 kHz

Input Attenuation 	:  20 dB (or auto)

RBW 			:  10 kHz (or auto)

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Freq: [FREQUENCY] > [818] > {MHZ}

## Span: [SPAN] > [300] > {KHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [20] > {dB}

RBW: [BW/AVG] > {Res BW} > {10} > {KHZ}

OBSERVATIONS/DATA RECORDING

- What is the power level displayed on the SA?

LO level measured at SA, Psa_LO @ 818 MHz = -4.7dBm

[Use marker function – search to peak if necessary]

- Use the SA built−in noise marker to measure the phase noise:

Marker Function 	: Phase noise on

Offset 			: Manual

Offset value		: 100 kHz, 50 kHz and 10 kHz

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Place Marker @818MHz: [Peak Search]

## Marker: [Marker] > {Function} > {Phase Noise} > {Offset Manual} > [100] > {KHz}

Turn on Phase Noise Measurement: {Phase Noise On}

OBSERVATIONS/DATA RECORDING

- What is the phase noise displayed on the SA?

Phase noise @100 kHz offset = _______ dBc/Hz

- What is the phase noise displayed on the SA?

Phase noise @50 kHz offset = _______ dBc/Hz

- What is the phase noise displayed on the SA?

Phase noise @10 kHz offset = _______ dBc/Hz

Reference

Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics”, Keysight Technologies, 2001