ME1000 RF Circuit Design

Lab 10

Mixer Characterization using Spectrum Analyzer (SA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objectives:

- To measure some important characteristics of the frequency mixer with a SA.

- To understand the frequency conversion principles of the frequency mixer.

Equipments and Accessories Required:

- Spectrum analyzer

- Signal generator

- ME1000 Receiver unit

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 10 – Mixer Characterization using SA

Basic Equipment Setup

Required accessories

SMA m-m coaxial cable

USB cable

PC/Notebook with RF Trainer Control Panel installed

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

Spectrum Analyzer

RF Input

CW Signal Generator

RF Output

SMA cable

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

RF In @ 868 MHz

LO In

@ 818 MHz

SMA jumper cable

IF out @ 50 MHz

Receiver unit

Figure 1 General equipment configuration for mixer measurement using SA

Determine the local oscillator (LO) input level using a spectrum analyzer

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

Spectrum Analyzer

RF Input

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Plo

Psa_LO

Figure 2 Equipment set up for LO input level determination

We must determine the input LO power level to the mixer by connecting the LO directly to the SA as shown in Figure 2.

- Connect the output of the frequency synthesizer module directly to the SA.

- On PC/Notebook, launch the RF Trainer Control Panel. Select RX Unit from the board selection and click “Connect to RF Trainer”. In the Frequency Synthesizer Control area, click “Frequency Synthesizer Off” to power up the synthesizer. Set the frequency to 818MHz. You will see the frequency synthesizer PCB LED light up on the trainer, and the button text change to “Frequency Synthesizer On”.

- Set the following settings for SA:

Centre Frequency	:  818 MHz

Span			:  10 MHz

RBW			:  100 kHz

Attenuation		:  30 dB (or auto)

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Preset to default settings: [Preset/System] > {Preset}

## Center Freq: [FREQUENCY] > [818] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [30] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

OBSERVATIONS/DATA RECORDING

- What is the power level displayed on the SA when LO is power on?

LO level measured at SA, Psa_LO @818 MHz = −3.725 dBm

[Use marker function – search peak if necessary]

- Determine the actual output power level of the LO.

Actual LO output power, Plo = Psa_LO + Loutput_path

= −3.725 dBm + 1.025 dB = −2.7 dBm

where,

Loutput_path = Lcable_sa + Lhalf_path (cable/connector/PCB trace losses found previously)

Conversion loss measurement

Take note of the following notation used in mixer measurement:

RF

LO

IF

SG

SA

Psig_gen

(SG input power)

Plo

(LO output power)

Pin_RF

Pin_LO

Pout_RF

Pout_IF

Pout_LO

Pout_sa_RF

Pout_sa_IF

Pout_sa_LO

Pin_RF : Input RF power into mixer’s RF port

Pin_LO: Input LO power into mixer’s LO port

Pout_RF : Output RF power from mixer’s IF port

Pout_IF : Output IF power from mixer’s IF port

Pout_LO : Output LO power from mixer’s IF port

Pout_sa_RF : Output RF power from mixer’s IF port measured at SA

Pout_sa_IF: Output IF power from mixer’s IF port measured at SA

Pout_sa_LO : Output LO power from mixer’s IF port measured at SA

Figure 3. Notation used in mixer measurement

- Make the connection as shown in Figure 1. Similarly, power on the mixer by clicking “Mixer Off”. You will see the mixer PCB LED light up on the trainer, and the button text change to “Mixer On”.

- On the RF Trainer Control Panel, click “Connect to RF Trainer”. In the Frequency Synthesizer Control area, click “Frequency Synthesizer Off” to power up the synthesizer. Set the frequency to 818MHz. You will see the frequency synthesizer PCB LED light up on the trainer, and the button text change to “Frequency Synthesizer On”.

- Use the following settings for signal generator (SG):

CW Frequency 	:  868 MHz

Power		 	:  –15 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [15] > {dBm}

Turn on output: [RF On/Off]

- Use the following settings for SA:

Start Frequency 	:  10 MHz

Stop Frequency 	:  900 MHz

Input Attenuation 	:  20 dB (or auto)

RBW 			:  100 kHz (or auto)

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Start Freq: [FREQUENCY] > {Start Freq} > [10] > {MHZ}

## Stop Freq: {Stop Freq} > [900] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [20] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

OBSERVATIONS/DATA RECORDING

- Plot the mixer output spectrum from 10 MHz to 900 MHz in the following graph by identifying all the origin and converted tones (such as LO, RF or RF ± LO) and their respective output power levels.

50 MHz								   	     818 MHz 868 MHz

Output RF power measured at SA @868 MHz, Pout_sa_RF = −30.66 dBm

Output LO power measured at SA @818 MHz, Pout_sa_LO = −20.08 dBm

Output IF power measured at SA @50 MHz, Pout_sa_IF = −16.48 dBm

- Determine the actual input and output RF power. Use Linput_path and Loutput_path from the previous lab. Input RF power from the SG, Psig_gen = −15 dBm

Actual RF input power into the mixer, Pin_RF = Psig_gen − Linput_path @868 MHz

= −15 dBm − (−1.165) dB = −16.165 dBm

RF output power measured at the SA, Pout_sa_RF = −30.66 dBm

Actual RF output power from the mixer, Pout_RF = Pout_sa_RF + Loutput_path @868 MHz

= −30.66 dBm + 1.185 dB = −29.475 dBm

- Determine the actual IF output power. Use Lcable from previous lab.

IF output power measured at the SA, Pout_sa_IF = −16.48 dBm

Actual IF output power from the mixer, Pout_IF = Pout_sa_IF + Loutput_path @50 MHz

= −16.48 dBm + 0.22 dB = −16.26 dBm

- Determine the actual LO input and output power. Find the jumper cable loss which connects LO_in to RF_out, Ljumper as in Lab 1.

LO input power measured previously, Plo = −2.7 dBm

Actual LO input power into the mixer, Pin_LO = Plo − Ljumper @818 MHz

= −2.7 dBm − 0.985 dB = −3.685 dBm

LO output power measured at the SA, Pout_sa_LO = −20.08 dBm

Actual LO output power, Pout_LO = Pout_sa_LO + Loutput_path @818 MHz

= −20.08 dBm + 1.025 dB = −19.055 dBm

- Calculate the conversion loss of the mixer.

Conversion loss = Actual input RF power (dBm) – Actual output IF power (dBm)

= Pin_RF − Pout_IF

= −16.165 dBm − (−16.26) dBm  = 0.095 dB

- Calculate the RF-to-IF isolation of the mixer.

RF-to-IF isolation = Actual input RF power (dBm) – Actual output RF power (dBm)

= Pin_RF − Pout_RF

= −16.165 dBm – (−29.475) dBm = 13.31 dB

- Calculate the LO-to-IF isolation of the mixer.

LO-to-IF isolation = Actual LO input power (dBm) – Actual LO output power (dBm)

= Pin_LO − Pout_LO = −3.685 dBm − (−19.055) dBm = 15.37 dB

Gain compression measurements

- Use the same setup as shown in Figure 1.

- Use the following settings for SG:

CW Frequency 	:  868 MHz

Power		 	:  –15 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [868] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [15] > {dBm}

Turn on output: [RF On/Off]

- Use the following setting for SA:

Centre Frequency 	:  50 MHz

Span 		 	:  10 MHz

Input Attenuation 	:  30 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Freq: [FREQUENCY] > [50] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} >[30] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

- Determine the actual IF output power levels at different RF input power levels.

OBSERVATIONS/DATA RECORDING

- Fill in the table below. Use Linput_path (dB) and Loutput_path (dB) found in the previous lab.

| SG input power, Psig_gen (dBm) | Actual RF input power, Pin_RF (dBm) | IF output power measured at SA, Pout_sa_IF (dBm) | Actual IF output power, Pout_IF (dBm) |
| --- | --- | --- | --- |
| −15 | −16.165 | −16.45 | −15.265 |
| −14 | −15.165 | −15.51 | −14.325 |
| −13 | −14.165 | −14.59 | −13.405 |
| −12 | −13.165 | −13.7 | −12.515 |
| −11 | −12.165 | −12.84 | −11.655 |
| −10 | −11.165 | −12 | −10.815 |
| −9 | −10.165 | −11.2 | −10.015 |
| −8 | −9.165 | −10.46 | −9.275 |
| −7 | −8.165 | −9.772 | −8.557 |
| −6 | −7.165 | −9.146 | −7.905 |
| −5 | −6.165 | −8.602 | −7.337 |
| −4 | −5.165 | −8.116 | −6.837 |
| −3 | −4.165 | −7.724 | −6.451 |
| −2 | −3.165 | −7.452 | −6.267 |
| −1 | −2.165 | −7.368 | −6.057 |
| 0 | −1.165 | −7.276 | −6.047 |

- Plot Pout_IF as a function of Pin_RF in the graph above.

- At what output power level is the gain decreased by 1 dB compared with the gain at a very low input power level?

P1dB output = −8.7 dBm

P1dB input = Output P1dB output – Gain = −5.2 dBm

Image frequency rejection

Image frequency is an undesired input frequency in a mixer that produces the same IF as the desired input signal. An image must be rejected prior to mixing because once it has entered the IF chain, the image will be indistinguishable from the desired signal and impossible to filter out. Image rejection is accomplished by placing the necessary filter before the mixer.

LO

818 MHz

RF

868 MHz

Image

768 MHz

f

V

IF

50 MHz

Image = RF – 2IF

IF

50 MHz

Figure 4 Image frequency illustration

- Use the same setup as described previously.

- Change the settings for the SG to explore the effect of image frequency.

CW Frequency  	:  768 MHz

Power level 		:  –15 dBm

###### N9310A setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [Frequency] > [768] > {MHZ}

## Amplitude: [Amplitude] > [+/-] > [15] > {dBm}

Turn on output: [RF On/Off]

- Change the settings for SA to explore the effect of image frequency.

Start Frequency 	:  10 MHz

Stop Frequency 	:  900 MHz

Input Attenuation 	:  20 dB (or auto)

RBW 			:  100 kHz (or auto)

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Start Freq: [FREQUENCY] > {Start Freq} > [10] > {MHZ}

## Stop Freq: {Stop Freq} > [900] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [20] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

OBSERVATIONS/DATA RECORDING

- Plot the mixer output spectrum from 10 MHz to 900 MHz in the following graph

4. Change to the following settings for the SA to determine the image frequency power level:

Centre Frequency 	:  50 MHz

Span		 	:  10 MHz

Input Attenuation 	:  30 dB (or auto)

RBW 			:  100 kHz (or auto)

Averaging		:  On

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

## Center Freq: [FREQUENCY] > [50] > {MHZ}

## Span: [SPAN] > [10] > {MHZ}

## Attenuation: [AMPLITUDE] > {Attenuation} > [30] > {dB}

RBW: [BW/AVG] > {Res BW} > {100} > {KHZ}

Averaging: [BW/AVG] > {Average}

OBSERVATIONS/DATA RECORDING

- What is the effect at the IF frequency if an image frequency entered the mixer?

The IF frequency will have TWO frequencies overlapping and they are NOT separable.

- What is the power level of the image frequency displayed on the SA?

Image frequency @50 MHz, Pimage = −12.8dBm

[Use marker function – search peak if necessary]

- How do we reduce the image problem?

Connect a band-pass filter to the RF input, with the stop-band corresponds to the image frequency.

- Connect the RF band-pass filter module before the mixer and determine the power level of the image frequency displayed on SA.

Image frequency @50 MHz after filtering, Pimage_filter = −35.4 dBm

- Calculate the image rejection provided by this filter.

Image rejection = Pimage_filter − Pimage

= −35.4 dBm − (−12.8) dBm

= −22.6 dB

References

- Presentation slides, “A Seminar on RF Measurement – Spectrum Analysis Basics”, Keysight Technologies, 2001

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.

- David M. Pozar, “Microwave Engineering”, 3rd Edition, John Wiley & Sons, 2005.

Image

768 MHz