ME1000 RF Circuit Design

Lab 16

End-to-end RF Transceiver Measurement

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

i.	To analyze signal flow through each module in the transceiver chain.

Equipments and Accessories Required:

- Spectrum analyzer

- Signal generator

- ME1000 Transmitter unit

- ME1000 Receiver unit

- Antennas

- SMA m-m coaxial cables

- USB cables

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 16 – End-to-end RF Transceiver Measurement with SA

PC/

Notebook

PC/

Notebook

Spectrum Analyzer

RF Input

USB cable

USB cable

RF antenna

SMA cable

SMA cable

Transmitter unit

Receiver unit

CW Signal Generator

RF Output

Basic Equipment Setup

Required accessories

PC/Notebook with RF Trainer Control Panel installed

SMA m-m coaxial cable

USB cable

Figure 1 General equipment configuration for end-to-end transceiver unit measurement using a SA

Installing the RF Trainer Control Panel Software

- Run the “setup.exe” from the supplied CD if you have not installed the RF Trainer Control Panel software.

- Start two control panels by clicking the RF_Trainer icon on your PC desktop twice.  One control panel will control the transmitter unit while the other will control the receiver unit.

### Part 1: Transmitter unit setup

- Connect the PC to the transmitter unit via the USB cable.

- Select TX Unit from one of the two control panels and click “Connect to RF Trainer”. The green LED will blink once on the Control Unit of the transmitter unit indicating a successful connection.

| ## Verification of the transmitter unit |  |
| --- | --- |
| 1 Click on “Frequency Synthesizer Off” to power up the frequency synthesizer. Set frequency to 818 MHz. | TX Control Panel 1 |
| Synthesizer verification 2 Connect the spectrum analyzer to the Synthesizer Out port. 3 Check if the output signal from the synthesizer is correct. | Transmitter unit’s Synthesizer Out port Spectrum analyzer 2 3 N9320A/B Spectrum Analyzer setting: Preset to default settings: [Preset/System] > {Preset} #### Center Freq: 818 MHz [Frequency] > {Center Freq} > [818] > {MHZ} #### Span: 10MHz [SPAN] > [10] > {MHZ} |
| 4 Click on “Up-Converter Off” to power up the mixer. |  |
| N9310A Signal Generator setting: #### Frequency: 50MHz [Frequency] > [50] > {MHZ} Amplitude: -15 dBm [Amplitude] > [+/-] > [15] > {dBm} Turn on output: [RF On/Off] 5 Reconnect the spectrum analyzer to IF Out port. 6 Use the jumper cable to connect the Synthesizer Out to Mixer LO In. 7 Connect the signal generator (50 MHz, −15 dBm) to the Mixer RF In port. 8 Check the spectrum analyzer to see if the mixer produces any intermodulation products i.e. (±n818 ± m50) MHz signals. | N9320A/B Spectrum Analyzer setting: Center Freq: 818 MHz Span: 200 MHz [SPAN] > [200] > {MHZ} 8 Spectrum analyzer Signal generator 7 50 MHz -15 dBm 5 6 818 MHz |
| End-to-End transmitter verification 9 Reconnect the spectrum analyzer to the Bandpass Filter Out port. 101 Use the jumper cables to connect all ports as shown. 112 Click “Power Amplifier Off” on the RF Trainer Control Panel to power up the power amplifier. 12 Check the spectrum analyzer to see if the signal generator output is upconverted from 50 MHz to 868 MHz. | N9320A/B Spectrum Analyzer setting: Center Freq: 868 MHz Span: 20 MHz N9310A Signal Generator setting: Frequency: 50 MHz Amplitude: −15 dBm Signal Generator Spectrum Analyzer 12 868 MHz 50 MHz -15 dBm 9 10 818 MHz |

OBSERVATIONS/DATA RECORDING

- Vary the frequency setting of the frequency synthesizer and observe the signal output on the spectrum analyzer. Does the 50 MHz input signal upconvert accordingly?  Explain your observation.

To capture the frequency spectrum on the spectrum analyzer while varying the frequency synthesizer setting, setup the spectrum analyzer as follows:

N9320A setup: [View/Trace] > [Max Hold]

This setting will store the display of all previous peak signals captured.

The spectrum analyzer should capture frequency value = 50 MHz + synthesizer frequency setting.

- Vary the synthesizer setting from the lowest to the highest (using the up/down button) and observe the frequency spectrum. Pay attention to the RF signal which is lower than the others.

Use the marker to measure the amplitude. Press [Peak Search] and for the softkey, press either {Next Peak}, {Next Pk Right} or {Next Pk Left}.

Determine the 3 dB bandwidth of the transmitter from the spectrum captured on the spectrum analyzer.

Place a marker on the signal (at the lower frequency edge) which is 3 dB lower than the highest peak on the spectrum by turning the front-panel knob. Then, press [Marker] > {Delta} > {Delta On}. This will anchor the first marker and activate a second marker. Turn the knob again to move the 2nd marker to another signal (at the upper frequency edge) which is also 3 dB lower than the highest peak on the spectrum.

Lower 3 dB frequency = 867.65 MHz

Upper 3 dB frequency = 869.35 MHz

3 dB bandwidth 	= 1.7 MHz

### Part 2: Receiver unit setup

- Connect the PC to the receiver unit via the USB cable. (Note: the same PC can be used to make this second connection)

- Select RX Unit from the other control panel and click “Connect to RF Trainer”. The green LED will blink once on the Control Unit of the transmitter unit indicating a successful connection.

| Verification of receiver unit |  |
| --- | --- |
| 1 Click “Frequency Synthesizer Off” to power up the frequency synthesizer. Set frequency to 818 MHz. | 1 RX control panel |
| Synthesizer verification 2 Connect the spectrum analyzer to the Synthesizer Out port. 3 Check if the output signal from the synthesizer is correct. | Receiver unit’s Synthesizer Out port Spectrum analyzer 2 3 |
| Mixer verification 4 Click on “Mixer Off” to power up the Mixer. |  |

| 5 Reconnect the spectrum analyzer to IF Out port. 6 Use a jumper cable to connect the Synthesizer Out to Mixer LO In port. 7 Reconnect the signal generator (868 MHz, −15 dBm) to Mixer RF In port. 8 Check the spectrum analyzer to see if the mixer produces intermodulation products, i.e. (±n818 ± m868) MHz signals. | Spectrum analyzer Signal generator 8 868 MHz -15 dBm 5 7 818 MHz 6 |
| --- | --- |
| End-to-End receiver verification 9 Reconnect the spectrum analyzer to the Band-pass Filter Out port. 10 Reconnect the signal generator to the Band-pass Filter In port. 11 Use jumper cables to connect all the ports shown. 12 Click “LNA Off” and “IF Amplifier Off” to power up the LNA and IF amplifier. 13 Check the spectrum analyzer to see if the signal generator output is downconverted from 868 MHz to 50 MHz. Note: Adjust the IF amplifier gain to capture a larger signal on the SA. | Spectrum analyzer Signal generator 13 14 868 MHz -15 dBm 11 50 MHz 9 10 818 MHz 12 12 |

OBSERVATIONS/DATA RECORDING

a.	Set the signal generator frequency to 867 MHz to simulate a different carrier. What do you need to do to get back the 50 MHz IF signal at the receiver output?

What are the advantages of a super-heterodyne receiver?

In order to get back the 50 MHz signal, switch the frequency synthesizer to 867 MHz − 50 MHz = 817 MHz.

Advantages of a super-heterodyne receiver are:

i. 	Only need to design the receiver circuit for one IF frequency

  - Easier to stabilize tunable oscillator than tunable filter (good Freq stability)

  - IF filter gives much narrower passband than RF filter (good selectivity)

- Reconnect the signal generator to the input of the LNA as shown below:

Set the signal generator frequency to 768 MHz. Do you get a 50 MHz signal on the spectrum analyzer? Explain your observation.

With an 818 MHz local synthesizer, the IF is still 50MHz – this is due to aliasing.

- What is this 768 MHz signal called? Explain the problem faced with the presence of this signal and how this problem is avoided in the receiver unit architecture.

This 768MHz signal is called image frequency – it is an undesired input frequency in a mixer that produces the same IF as the desired input signal. An image must be rejected prior to mixing because once it has entered the IF chain, the image will be indistinguishable from the desired signal and impossible to filter out.

LO

818 MHz

RF

868 MHz

Image

768 MHz

f

V

IF

50 MHz

Image = RF – 2IF

IF

50 MHz

##### With a band-pass filter that rejects the image frequency being placed before the mixer, the problem is resolved.

##### Part 3: End-to-end RF transceiver setup

1 2 0.5 m 3

1 Connect the antenna to the transmitter unit. 2 Connect the antenna to the receiver unit. 3 Separate the units to about 0.5 m meter apart.

5 868 MHz 50 MHz -15 dBm 50 MHz 4 818 MHz 6

4 Reconnect and power up the signal generator as shown. 5 Check the spectrum analyzer to see if the signal generator output from the transmitter unit is received at the receiver unit. To do this, you have to power up the upconverter, power amplifier, frequency synthesizer and set it at 818 MHz on the transmitter unit as well as the LNA, mixer, IF amplifier, frequency synthesizer and set it at 818 MHz on the receiver unit. 6 The received signal is in the region of −50 dBm. Ensure the noise floor of your spectrum analyzer is below this level. You might need to change the IF Amplifier gain (for example to Level 8) if the received signal is too weak or your spectrum analyzer’s noise floor is too high.

OBSERVATIONS/DATA RECORDING

  - Observe the presence of signal on the spectrum analyzer, use the peak search function to measure the signal frequency.

(If the signal is too low, move either the transmitter and receiver unit closer to each other or increase the IF amplifier gain setting)

A −48 dBm signal at 50 MHz is observed.

b.	Set the spectrum analyzer center frequency to 50 MHz, span to 2 MHz, resolution bandwidth to 40 KHz.

Center Frequency 		: 50 MHz

Span				: 2 MHz

Resolution Bandwidth 	: 40 kHz

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Center Frequency: [FREQUENCY] > {Center Freq} > [50] > [MHZ]

Span: [SPAN] > [2] > [MHZ]

RBW: [BW/AVG] > [40] > [KHZ]

Set the signal generator to 50 MHz and turn on the frequency modulation (FM) with the following setting:

FM deviation 	: 80 KHz

FM rate 	: 80 KHz

FM Source 	: Internal

N9320A/B setting: “[ ]”: Hardkey; “{ }”: Softkey

Frequency: [FREQUENCY] > [50] > [MHZ]

Modulation: [Mod On] > [Mod On/Off]

FM Deviation: [FM] > {FM Dev} > [80] > [KHZ]

FM Rate: {FM Rate} > [80] > [KHZ]

FM Source: {FM Source} > {Internal}

FM Modulation: {FM On} > [FM On/Off]

Determine the modulating frequency from the spectrum displayed on the spectrum analyzer.

Carrier frequency

Modulating frequency

The carrier frequency found to be at 50.02 MHz and the modulating frequency is 80 kHz.

- Reduce the FM rate to 70 kHz and observe the change on the spectrum analyzer.  Increase the resolution bandwidth of the spectrum analyzer to 100 kHz. Are you still able to measure the modulating frequency? Why?

When the FM rate decreases to 70 kHz, the modulating frequency also drops to 70 kHz. When the spectrum analyzer resolution bandwidth setting is larger than the spacing between the two signals, the analyzer sees them as one signal. With this, the modulating frequency could not be measured. To resolve two equal amplitude signals, the resolution bandwidth setting has to be at least equal or smaller. For two unequal amplitude signals, the resolution bandwidth setting has to be much lower.