ME1000 RF Circuit Design

Lab 5

Mixer Characterization using

Vector Network Analyzer (VNA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

- To measure SWR, return loss, RF feedthrough and LO isolations of a mixer with VNA.

Equipments and Accessories Required:

- Vector network analyzer

- ME1000 Receiver unit

- TRM calibration board/Matched load (50Ω)

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 11 – Mixer Characterization using VNA

Basic Equipment Setup

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

SMA cable

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

SMA jumper cable

Receiver unit

Vector Network Analyzer

P1

P2

Mixer Output

(IF)

Mixer Input

(RF)

Synthesizer Output

Required accessories

SMA m-m coaxial cable

USB cable

PC/Notebook with RF Trainer Control Panel installed

SMA m-m jumper cable

Match PCB board (50 Ω)

SMA m-m adapter

match

Figure 1 General equipment configuration for mixer measurement using VNA

IMPORTANT:

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall>. Using the knob, select the saved calibration data in the non-volatile memory and finally, press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, the <Scale>, <Format> and <Marker> menus are the most helpful.  Under <Scale>, the AUTO SCALE option can be quite handy, but you will likely use it less and less as you get more comfortable with manipulating the display.

Finally, if you hit <Preset> your calibration will need to be recalled. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM thru PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “calibration kit” selection to the default kit that generally does not correspond to the TRM PCB boards you will use)

***Note: Use the marker to record down your readings. Marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for max and min points.

RF/IF port SWR measurement with VNA

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

SMA jumper cable

Receiver unit

Vector Network Analyzer

P1

P2

Mixer input (RF)

Synthesizer output

SMA adapter

Matched load

Figure 2 RF port SWR measurement with VNA

Make the connection as shown in Figure 2 to determine the RF port SWR. Note that you must connect the synthesizer output into the LO port of the mixer via a SMA jumper cable. Also, terminate the IF port with the 50 Ω matched PCB board. Port 2 of VNA is not use.

- Recall the TRM calibration data (saved in the previous lab) from the non-volatile memory.

- On the PC/Notebook, launch the RF Trainer Control Panel. Click Connect to RF Trainer button. Under the RX Unit Control area, click the “Mixer Off” to power up the mixer. You will see the mixer PCB LED light up on the trainer, and the button text change to “Mixer On”. Next, under the Frequency Synthesizer Control area, click “Frequency Synthesizer Off” to power up the synthesizer and set the frequency to 818MHz.

- Use the following settings to determine the SWR of the RF port:

Start		:  700 MHz

Stop		:  900 MHz

Format 	:  SWR

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey ; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [900] > [M/μ]

Format: [Format] > {SWR}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at 868 MHz?

SWR @ 868 MHz = 1: 5.29

- Change the setting to display the log magnitude.

Format 	:  Log magnitude

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the input return loss at 868 MHz?

Input return loss @868 MHz = −4.5 dB

- Plot the input return loss from 700 MHz to 900 MHz.

- Connect the IF port to port 1 of VNA and terminate the RF port with the matched load. Change the start and stop frequencies to 10 MHz and 100 MHz respectively.  Make sure that you have turned off the mixer DC supply before removing any connections by clicking “Mixer Off” in the RF Trainer Control Panel software.  Power on the mixer again when the proper connection has been made.

Start		:  10 MHz

Stop		:  100 MHz

Format 	:  SWR

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [10] > [M/μ]

Stop Frequency: [Stop] > [100] > [M/μ]

Format: [Format] > {SWR}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at 50 MHz?

SWR @50 MHz = 1: 4

- Change the setting to display the log magnitude.

Format 	:  log magnitude

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the input return loss at 50 MHz?

Input return loss @50 MHz = −4.3 dB

- Plot the input return loss from 10 MHz to 100 MHz.

LO port SWR measurement with VNA

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Vector Network Analyzer

P1

P2

SMA adapter

Matched Load

SMA adapter

Figure 3 LO port SWR measurement with VNA

Make the connection as shown in Figure 3 to determine the LO port SWR. Note that you need to terminate both the IF and RF ports with a 50 Ω matched PCB board. Port 2 of the VNA and frequency synthesizer are not use in this measurement.

- On PC/Notebook, launch the RF Trainer Control Panel. Click Connect to RF Trainer. Under the RX Unit Control area, click “Mixer Off” to power up the mixer. You will see the mixer PCB LED light up on the trainer, and the button text change to “Mixer On”.

- Use the following settings to determine the SWR of the RF port:

Start		:  700 MHz

Stop		:  900 MHz

Format 	:  SWR

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [900] > [M/μ]

Format: [Format] > {SWR}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at 818 MHz?

SWR @818 MHz = 1: 5.5

- Change the setting to display the log magnitude plot.

Format 	:  Log magnitude

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the input return loss at 818 MHz?

Input return loss @818 MHz = −4.7 dB

- Plot the input return loss from 700MHz to 900MHz.

RF feedthrough measurement

Make the connection as shown in Figure 1 to determine the RF feedthrough. Note that you must connect the synthesizer output into the LO port of the mixer via a SMA jumper cable. Also, connect both the RF and IF ports to port 1 and port 2 of the VNA.

- On PC/Notebook, launch the RF Trainer Control Panel. Click Connect to RF Trainer. Under the RX Unit Control area, click “Mixer Off” to power up the mixer. You will see the LNA PCB LED light up on the trainer, and the button text change to “LNA On”. Next, under the Frequency Synthesizer Control area, adjust the frequency to 818MHz. This is the LO frequency input into the mixer.

- Use the following settings:

Start		:  700 MHz

Stop		:  900 MHz

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [900] > [M/μ]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the RF feedthrough at 868 MHz.

RF feedthrough @868 MHz = −15.5dB

- Plot the RF feedthrough from input frequency of 700 MHz to 900 MHz.

LO−to−RF isolation measurement

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Receiver unit

Vector Network Analyzer

P1

P2

SMA Adapter

Matched Load

Figure 4 LO−to−RF isolation measurement with VNA

Make the connection as shown in Figure 4 to determine the LO−to−RF isolation. Connect both the LO and RF ports to port 1 and port 2 of the VNA respectively. The IF port is terminated with a 50 Ω matched load.

- On PC/Notebook, launch the RF Trainer Control Panel. Click Connect to RF Trainer. Under the RX Unit Control area, click “Mixer Off” to power up the mixer. You will see the mixer PCB LED light up on the trainer, and the button text change to “Mixer On”.

- Use the following settings:

Start		:  700 MHz

Stop		:  900 MHz

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [900] > [M/μ]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the LO−to−RF isolation at frequency of 818 MHz.

LO−to−RF isolation @818 MHz = −6.5 dB

- Plot the isolation from input frequency of 700 MHz to 900 MHz.

LO−to−IF isolation measurement

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

In

Out

Out

PC/

Notebook

USB cable

USB port

Receiver unit

Vector Network Analyzer

P2

P1

SMA Adapter

Matched load

Figure 5 LO−to−IF isolation measurement with VNA

Make the connection as shown in Figure 5 to determine the LO−to−IF isolation. Connect both the LO and IF ports to port 1 and port 2 of the VNA respectively. The RF port is terminated with a 50 Ω matched load.

- On PC/Notebook, launch the RF Trainer Control Panel. Click Connect to RF Trainer. Under the RX Unit Control area, click “Mixer Off” to power up the mixer. You will see the mixer PCB LED light up on the trainer, and the button text change to “Mixer On”.

- Use the following settings:

Start		:  700 MHz

Stop		:  900 MHz

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [900] > [M/μ]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the LO−to−IF isolation at 818 MHz.

LO−to−IF isolation @818 MHz = −15.5 dB

- Plot the isolation from input frequency of 700 MHz to 900 MHz.

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Product Note 8753-2, “RF Component Measurements: Mixer Measurements Using the Keysight 8753B Network Analyzer”, Keysight Technologies.

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.