ME1000 RF Circuit Design

Lab 4

Filter Characterization using

Vector Network Analyzer (VNA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objectives:

- To characterize the RF filter with reflection and transmission measurement via a VNA.

- To display results in multiple plots such as Smith Chart, Magnitude and Phase plot.

Equipments and Accessories Required:

- Vector network analyzer

- ME1000 Transmitter unit

- SMA m-m coaxial cables

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 9 – Filter Characterization using VNA

Basic Equipment Setup

SMA m-m coaxial cable

RF band-pass filter

Power

amplifier

Up converter

Frequency synthesizer

RF Out

LO In

RF In

IF Out

In

Out

Out

In

USB port

Transmitter unit

Vector Network Analyzer

P1

P2

SMA Cable

SMA Cable

Required Accessories

Figure 1 General equipment configuration for filter measurement

IMPORTANT:

There are multiple measurement channels on the VNA which can be thought of as having multiple VNAs in one equipment. You might want to use Channel 1 for transmission measurements and Channel 2 for reflection measurements.

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall>. Using the knob, select the saved calibration data in the non-volatile memory and finally, press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, the <Scale>, <Format> and <Marker> menus are the most helpful.  Under <Scale>, the AUTO SCALE option can be quite handy, but you will likely use it less and less as you get more comfortable with manipulating the display.

Finally, if you hit <Preset> your calibration will need to be recalled. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM thru PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “calibration kit” selection to the default kit that generally does not correspond to the TRM PCB boards you will use)

***Note: Use the marker to record down your readings. Marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for max and min points.

Transmission Measurements

- Recall the calibration data (saved in previous lab) from the non-volatile memory.

- Connect the filter as shown in Figure 1.

- Use the following setting to determine the insertion loss of the filter:

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [500] > [M/μ]

Stop Frequency: [Stop] > [1.5] > [G/n]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

**Note: Set the start stop frequency till you see the filter function (between 300MHz and 1.5GHz range)

OBSERVATIONS/DATA RECORDING

a) 	What is the type of this filter? (band-pass, low-pass, high-pass, or not sure)

Band-pass filter.

b) 	Use the marker function to find the following parameters:

3dB cutoff frequencies (use minimum loss as the reference value):

Lower 3dB frequency (fL) = 825 MHz

Upper 3dB frequency (fH) = 1250 MHz

3dB bandwidth = 425 MHz

Centre frequency (fc) = 1037.5 MHz

[Note: Centre frequency is defined as mean of the lower and upper 3dB frequencies]

Insertion loss (S21) at centre frequency = −2.5 dB

[Note: The insertion loss should be −3 dB or less at center frequency]

- What is the importance of insertion loss?

A small insertion loss in the passband minimizes unnecessary signal attenuation. Moreover, the insertion contributes to increase in noise figure if this filter is used in a wireless receiver.

- What is the importance of insertion loss flatness?

The insertion loss flatness indicates the linearity of the filter. A very linear filter should have a better flatness.

- Explain why the S21 magnitude increases slowly in the upper stopband with frequency.

This is called RF leakage, could be due to (1) parasitic capacitance, e.g. electric field coupling between the Input and Output transmission lines via the surrounding metallic structures. (2) the properties of the discrete components which deteriorates with frequency.

- Change to the following setting to determine the group delay of the filter:

Format		:  Delay

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Group Delay}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING (The solutions for Steps 4 & 5 are only applicable to Vector Network Analyzer Version Lab)

- Find the group delay at the centre frequency.

Group delay (at fc) = 1.88 ns

- Is the group delay constant across the pass-band?

Somewhat constant, as the group delay ranges from 1.88 ns to 3.5 ns.

- How would you specify this filter in term of group delay across the pass-band?

Group delay across the pass-band = ( 1.88 ± 1.7) ns

- What information is provided by group delay?

It provides the transit time taken by a signal to travel from the input to the output of the filter.

- What is the importance of group delay flatness?

Group delay corresponds to the time electrical signal takes to go from input to the output of a 2-terminal component for that particular frequency where group delay is measured. A network with flat or uniform group delay across the frequency range of interest ensures minimal distortion of a wideband signal that consists of many frequency components within the frequency range of interest.

- Change to the following settings to display the phase of the filter:

Format		:  Phase

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Phase}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Sketch the phase response of the filter:

- How does the general shape of the phase response correlate with the group delay measured in the previous step?

Group delay  is given by: τ=−dφdω where  is the phase and  is frequency in radian/seconds.  When the phase variation with frequency is not a straight line (e.g. linear), it means group delay varies as a function of frequency.

- Change to the following settings to determine the out-of-band rejection:

Format		:  Log magnitude

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the rejection at stopband (at 300 MHz, 500 MHz and 800 MHz offset from the center frequency).

Upper rejection @300 MHz = −14.1 dB

Lower rejection @300 MHz = −15 dB

Upper rejection @500 MHz = −29.6 dB

Lower rejection @500 MHz = −37 dB

Upper rejection @800 MHz = −38 dB

Lower rejection @800 MHz = −57.8 dB

Reflection Measurements

- Use the following settings to determine the return loss of the filter:

Format 	:  Log magnitude

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the return loss at pass-band?

Return loss, S11 at pass-band = 13 dB at 868 MHz with the highest return loss of 23.5 dB occurs at 930 MHz.

- What is the return loss at stop-band?

Return loss, S11 at stop-band = 1 dB (at frequencies lower than 700 MHz and at frequencies higher than 1.4 GHz)

[Note: You can choose any frequency point within the pass-band or stop-band]

- What is the importance of return loss at the pass-band?

Return loss indicates the quality of the matching of the filter at pass-band with respect to the source. A large return loss (> 10 dB) is required for good matching at pass-band. A small return loss is required at stop-band for good rejection.

- From the return loss observed at pass-band and stop-band, describe the filter’s matching at both bands.

Pass-band – Good matching condition (all frequencies could pass)

Stop-band – Total mismatch condition (all frequencies rejected)

- Change the setting to display the Smith Chart for impedance measurement.

Format 	:  Smith Chart

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Smith} > {R+jX}

OBSERVATIONS/DATA RECORDING

- What is the impedance at passband?

Impedance at passband = 54.4 – j 24.4 Ω at 868 MHz, 56.8 + j 1.35 Ω at 930 MHz.

- What is the impedance at stopband?

Impedance at stopband = 5.49 – j 0.232 Ω

The impedance at stopband varies as it follows the edge of the Smith Chart. At different points in the Smith Chart plot, the impedance varies from having high values to low values.

[Note: You can choose any frequency point within the passband or stopband]

- What is the ideal impedance at passband?

Ideal Impedance at passband = 50 Ω

- What is the ideal impedance at stopband?

Ideal impedance at the stopband should be 0 Ω or anything that differs from c) by a huge amount.

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements”, Keysight Technologies.

- Application Note 1287-4, “Network Analyzer Measurements: Filter and Amplifier Examples”, Keysight Technologies.

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.