ME1000 RF Circuit Design

Lab 6

Antenna Reflection Measurement with

Vector Network Analyzer (VNA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

i. 	To characterize antenna with reflection measurement via VNA.

Equipments and Accessories Required:

- Vector network analyzer

- Antenna

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Lab 14 – Antenna Reflection Measurement with VNA

Vector Network Anal    r

P1

Basic Equipment Setup

Vector Network Analyzer

P1

P2

Antenna under measurement

(flat)

Antenna under measurement

(vertical)

(a)

(b)

Figure 1 General equipment configuration for antenna measurement

IMPORTANT:

There are multiple measurement channels on the VNA, which can be thought of as having multiple VNAs in one equipment. You might want to use Channel 1 for transmission measurements and Channel 2 for reflection measurements.

For this laboratory, you recall the calibration data from lab 1 by pressing <Save/Recall> and using the knob to select the saved calibration data in the non-volatile memory and finally pressing RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, the <Scale>, <Format> and <Marker> menus are the most helpful.  Under <Scale>, the AUTO SCALE option can be quite handy, but you will likely use it less and less as you get more comfortable with manipulating the display.

Finally, if you hit <Preset> your calibration will need to be recalled. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM thru PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “calibration kit” selection to the default kit that generally does not correspond to the TRM PCB boards you will use)

***Note: Use the marker to record down your readings. Marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for max and min points.

Reflection Measurements

- Recall the calibration data (saved in previous lab) from the non-volatile memory. If this setup is different from the previous one or no previous calibration was done, please perform reflection calibration (depends on whether S11 or S22 is used). It is recommended to connect the antenna directly to the VNA as this setup is able to hold the antenna at its fixed position.

For reflection calibration: Press “Cal”, “Calibrate”, then “S11” or “S22”. Connect the provided TRM short PCB board to the VNA port and then press “Short” to start the calibration. Alternatively, appropriate cal standards can be used to perform the required calibration.

- Connect the antenna flat as shown in Figure 1 (a).

- Use the following setting to determine the return loss of the antenna:

Format 	:  Log magnitude

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Plot S11 vs frequency. Use marker function to determine the minimum point.

Frequency (GHz)

Return Loss, S11 (dB)

Minimum return loss, S11 = −25.8 dB at 825 MHz, −11.4 dB at 868 MHz

- From the minimum return loss observed, describe the frequency/frequencies tuned by the antenna.

The frequency/frequencies tuned by the antenna is of the lowest return loss (S11).

- Using c = f , estimate the required length of the wired antenna on the tuned frequency. Does it correlate with the measured return loss? Explain your answer.

The might be some discrepancy due to the reflection observed during measurement of the return loss.

- Change the setting to display the SWR.

Format 	:  SWR

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {SWR}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at the tuned frequency?

SWR @tuned frequency = 1: 1.07

- Change the setting to display the Smith Chart plot for impedance measurement.

Format 	:  Smith Chart

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {SWR}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the impedance at the tuned frequency?

Impedance at tuned frequency = 45.5 + j 0.4 Ω

- What is the ideal impedance at the tuned frequency?

Ideal impedance at pass-band = 50 Ω

- Bent the antenna to vertical position as shown in Figure 1(b). Change the setting to display the return loss measurement.

Format 	:  Log magnitude

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Plot the S11 vs frequency. Use marker function to determine the minimum point.

Frequency (GHz)

Return Loss, S11 (dB)

Minimum return loss, S11 = −17.5 dB at 825 MHz, −8.5 dB at 868 MHz

- From the minimum return loss observed, describe the frequency/frequencies tuned by the antenna.

The frequency/frequencies tuned by the antenna is of the lowest return loss (S11).

- Are the results similar with the previous measurement where the antenna is in flat position? Comment and explain your answer.

No. This is because the strength of electromagnetic fields received in these two positions is different.

- Change the setting to display the SWR.

Format 	:  SWR

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {SWR}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at the tuned frequency?

SWR @tuned frequency = 1: 1.26

- Change the setting to display the Smith Chart plot for impedance measurement:

Format 	:  Smith Chart

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Smith} > {R+jX}

OBSERVATIONS/DATA RECORDING

- What is the impedance at the tuned frequency?

Impedance at tuned frequency = 41.4 + j 7.8 Ω

- What is the ideal impedance at the tuned frequency?

Ideal impedance at pass-band = 50 Ω

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements”, Keysight Technologies.