ME1000 RF Circuit Design

Lab 3

Low Noise Amplifier (LNA) Characterization

using Vector Network Analyzer (VNA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objective:

- To characterize LNA with reflection and transmission measurement via LNA.

Equipments and Accessories Required:

- Vector network analyzer

- ME1000 Receiver unit

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 6 – LNA Characterization using VNA

Basic Equipment Setup

LNA

Down

mixer

Frequency synthesizer

IF band-pass

filter

IF amplifier

LO In

RF Out

RF In

IF Out

Out

In

In

Out

Out

In

Required accessories

SMA m-m coaxial cable

USB cable

PC/Notebook with RF Trainer Control Panel installed

SMA cable

SMA cable

PC/

Notebook

RF Trainer Control Panel

USB cable

USB port

Vector Network Analyzer

P1

P2

Receiver unit

Figure 1 General equipment configuration for LNA measurement using a VNA

IMPORTANT INFORMATION:

In measuring a high gain amplifier, it is possible to overload the amplifier input. This will cause the unit to saturate, shows a significant non-linearity in the output signal and possibly damaging the unit. It can also damage the test equipment due to high power amplification. You should either add more attenuation to the input or output of the amplifier or reduce the RF power of the signal source before repeating the measurement.

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall>. Using the knob, select the saved calibration data in the non-volatile memory and finally, press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, the <Scale>, <Format> and <Marker> menus are the most helpful.  Under <Scale>, the AUTO SCALE option can be quite handy, but you will likely use it less and less as you get more comfortable with manipulating the display.

Finally, if you hit <Preset> your calibration will need to be recalled. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM thru PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “calibration kit” selection to the default kit that generally does not correspond to the TRM PCB boards you will use)

***Note: Use the marker to record down your readings. Marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for max and min points.

Transmission Measurements

- Recall the calibration data (saved in the previous lab) from the non-volatile memory.

- Connect the LNA as shown in Figure 1.

- Set the following settings to reduce the input source power to the LNA:

Power	 	:  Source power, –40 dBm

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Source Power: [Sweep Setup] > {Power} > {Power} > [± 40] > [x1]

- On PC/Notebook, launch the RF Trainer Control Panel. Select RX Unit from the board selection and click “Connect to RF Trainer”. In the RX Unit Control area, click “LNA Off” to power up the LNA. You will see the LNA PCB LED light up on the trainer, and the button text change to “LNA On”.

- Use the following settings to determine the small signal gain of the LNA for frequency range of between 700 MHz and 1 GHz:

Start		:  700 MHz

Stop		:  1 GHz

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [1] > [G/n]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the maximum gain, and the frequency at which it occurs?

Gmax = 12 dB at 700 MHz

- Find the frequency below this at which the gain is reduced by 3dB from the maximum gain.

Gmax – 3 dB occurs at 950 MHz.

- Find the gain at frequency of 868 MHz.

Gain @868 MHz = 10.2 dB

- Find the gain flatness.

Between 700 and 1 GHz, using the gain at 868 MHz as the nominal value:

Gain flatness = ± 1.835 dB

- Explain why the gain drops as the frequency increases.

The gain decreases with frequency due to (1) the presence of the small-signal capacitance between Base and Emitter, Ce, (2) the presence of small-signal capacitance between Base and Collector of the RF transistor, Cc,  (3) the presence of parasitic capacitance between the transistor package pins and (4) the properties of the other components such as the discrete capacitors, inductors and resistors which are frequency dependence.

- Is this gain a voltage or power gain? Explain.

Gain in RF usually means power gain.  See lecture notes.

- Change the following settings to display the phase of the LNA:

Format		:  Phase

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Phase}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Display the phase response. How does the general shape of the phase response compare to transmission phase measured in filter characterization?

Freq(GHz)

Phase

Phase is radian, it should be more or less a straight line. The slope corresponds to the group delay in seconds (if phase is in radian).

- Change the following settings to determine the group delay of the LNA:

Format		:  Delay

Scale		:  Autoscale

Avg		:  Averaging on

:  Smoothing on

:  Smoothing aperture (10%)

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Group Delay}

Scale: [Scale] > {Auto Scale}

Averaging: [Avg] > {Averaging ON}

Smoothing: {Smoothing ON}

Smoothing Aperture: {Smoothing Aperture} > [10] > [x 1]

OBSERVATIONS/DATA RECORDING

- Find the group delay at 868 MHz.

Group delay @868 MHz = 1.24ns

- Is the group delay constant across 700 MHz to 1 GHz?

YES, somewhat constant ranging from 0.94 ns to 1.28 ns.

- Find the group delay variation from 700 MHz to 1 GHz, using the group delay at 868 MHz as the nominal value.

The group delay variation is ±0.3 ns.

- Change the following settings to determine the isolation of the LNA:

Format		:  Log magnitude

Meas		:  S12

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S12}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the isolation 868 MHz.

Isolation @ 868 MHz = 18.8 dB

[Note: Use averaging, smoothing and smoothing aperture if necessary]

- Elaborate the importance of isolation in an amplifier.

An ideal amplifier would have infinite reverse isolation; no signal would be transmitted from the output back to the input. However, reflected signals can pass through the amplifier in the reverse direction. This unwanted reverse transmission can cause the reflected signals to interfere with the desired fundamental signal flowing in the forward direction. Therefore, reverse isolation is important to quantify.

Reflection Measurements

- Use the following settings to determine the input return loss of the LNA:

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the minimum input return loss and the frequency at which it occurs?

Min return loss = 6 dB at 1 GHz

- Find the maximum input return loss and the frequency at which it occurs?

Max return loss = 22 dB at 785 MHz

- Find the input return loss at frequency of 868 MHz.

Return loss @868 MHz = 15dB

[Note: Use the marker function to search for maximum and minimum]

- Change the settings to display the SWR.

Format 	:  SWR

E5061A Setting: “[ ]”: Front Panel Key; “{ }”: Softkey

Format: [Format] > {SWR}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at 868 MHz?

SWR @868 MHz = 1.49

- Change the settings to display the Smith Chart for input impedance measurement.

Format 	:  Smith Chart

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Smith} > {R+jX}

OBSERVATIONS/DATA RECORDING

a) 	What is the input impedance at 868 MHz?

Input Impedance @868 MHz = 72.3 − j 25.7 Ω

- What is the equivalent inductance (H) or capacitance (F) of the reactance?

Equivalent capacitance = 7.14 pF

- Explain the matching condition at the input.

To match the input for maximum power transfer, a source with impedance of 72.3 + j 25.7 Ω is needed.

- Use the following settings to determine the output return loss of the LNA:

Format 	:  Log magnitude

Meas		:  S22

Scale		:  Autoscale

E5061A Setting: “[ ]”: Front Panel Key; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S22}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the minimum output return loss and the frequency at which it occurs?

Min return loss = −18.3 dB at 776.5 MHz

- Find the maximum output return loss and the frequency at which it occurs?

Max return loss = −6.8 dB at 1 GHz

- Find the output return loss at 868 MHz.

Return loss @868 MHz = −13.2 dB

[Note: Use the marker function to search for maximum and minimum]

- Change the setting to display the SWR.

Format 	:  SWR

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {SWR}

OBSERVATIONS/DATA RECORDING

- What is the standing wave ratio at 868 MHz?

SWR @868 MHz = 1: 1.56

- Change the setting to display the smith chart for output impedance measurement.

Format 	:  Smith Chart

E5061A Setting: “[ ]”: Hardkey ; “{ }”: Softkey

Format: [Format] > {Smith} > {R+jX}

OBSERVATIONS/DATA RECORDING

a)   What is the output impedance at 868 MHz?

Output impedance@868 MHz = 76.7 + j 6.5 Ω

- What is the equivalent inductance (H) or capacitance (F) of the reactance?

Equivalent inductance = 1.2 nH

- Explain the matching condition at the output.

In theory, it should be close to 50 Ω.

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements”, Keysight Technologies.

- Application Note 1287-4, “Network Analyzer Measurements: Filter and Amplifier Examples”, Keysight Technologies.

- Product Note 8753-1, “RF Component Measurements: Amplifier Measurements Using the Keysight 8753 Network Analyzer”, Keysight Technologies.

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.