ME1000 RF Circuit Design

Lab 2

Power Amplifier (PA) Characterization

using Vector Network Analyzer (VNA)

____________________________________________________________________________

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this product may be reproduced, copied, or distributed in any form or by any means without expressed written consent from Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see Courseware Product License Agreement.

____________________________________________________________________________

Objectives:

- To characterize PA with reflection and transmission measurement via VNA.

- To measure gain compression using power sweeps via VNA.

Equipments and Accessories Required:

- Vector network analyzer

- ME1000 Transmitter unit

- SMA m-m coaxial cables

- USB cable

- PC/Notebook with RF Trainer Control Panel installed

DreamCatcherTM is the Trade Mark of Dream Catcher Consulting Sdn. Bhd.

Note: Turn off the transceiver trainer kit when not in use. The trainer will turn off automatically when no mouse or keyboard action is detected for more than 10 minutes. Always ensure that the casing is grounded to earth and the cover is latched up before powering up the device.

Lab 4 – Power Amplifier Characterization using VNA

Basic Equipment Setup

PC/

Notebook

RF Trainer Control Panel

USB cable

RF band-pass filter

Power

amplifier

Up converter

Frequency synthesizer

RF Out

LO In

RF In

IF Out

In

Out

Out

In

USB port

Transmitter unit

Vector Network Analyzer

P1

P2

SMA cable

SMA cable

Required accessories

SMA m-m coaxial cable

PC/Notebook with RF Trainer Control Panel installed

USB cable

Figure 1 General equipment configuration for PA measurement using VNA

IMPORTANT:

In measuring a high gain amplifier, it is possible to overload the amplifier input. This will cause the unit to saturate and a significant non-linearity in the output signal which will possibly damage the unit. It can also damage the test equipment due to high power amplification. You should either add more attenuation to the input or output of the amplifier or reduce the RF power of the signal source before repeating the measurement.

For this laboratory, you have to recall the calibration data from Lab 1 by pressing <Save/Recall>. Using the knob, select the saved calibration data in the non-volatile memory and finally, press RECALL STATE.

It is best to look at just one channel at a time. That is, when you are looking at transmission measurements, turn Ch 1 off, and when you are doing reflection measurements, turn Ch 2 off. When viewing signals, the <Scale>, <Format> and <Marker> menus are the most helpful.  Under <Scale>, the AUTO SCALE option can be quite handy, but you will likely use it less and less as you get more comfortable with manipulating the display.

Finally, if you hit <Preset> your calibration will need to be recalled. You can verify your calibration at anytime with a TRM short PCB board for reflection and a TRM thru PCB board for transmission. Similarly, the above calibration can be performed using appropriate cal standards. (Preset also sets the “calibration kit” selection to the default kit that generally does not correspond to the TRM PCB boards you will use)

***Note: Use the marker to record down your readings. Marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to look for max and min points.

Transmission Measurements

- Recall the calibration data (saved in the previous lab) from the non-volatile memory.

- Connect the PA as shown in Figure 1.

- Set the following settings to reduce the input source power to PA:

Power	 	:  Source power, –40dBm

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Source Power: [Sweep Setup] > {Power} > {Power} > [± 40] > [x1]

- On PC/Notebook, open the RF Trainer Control Panel. Select TX Unit from the board selection and click “Connect to RF Trainer”. In the TX Unit Control area, click on “Power Amplifier Off” to power up the PA. You will see the PA PCB LED light up on the trainer, and the button text change to “Power Amplifier On”.

- Use the following settings to determine the small signal gain of PA for frequency range of between 700 MHz and 1 GHz:

Start		:  700 MHz

Stop		:  1 GHz

Format 	:  Log magnitude

Meas		:  S21

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Start Frequency: [Start] > [700] > [M/μ]

Stop Frequency: [Stop] > [1] > [G/n]

Format: [Format] > {Log Mag}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the maximum gain, and the frequency at which it occurs?

Gmax = 16 dB at 824.5 MHz

- Find the frequency below this at which the gain is reduced by 3dB from the maximum gain.

Gmax –3dB occurs at 1025 MHz.

- Find the gain at 868 MHz.

Gain @868 MHz = 15.7 dB

- Change the following settings to determine the isolation of the PA:

Format		:  Log magnitude

Meas		:  S12

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S12}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the isolation at 868 MHz

Isolation @ 868 MHz = 22 dB

[Note: Use averaging, smoothing and smoothing aperture if necessary]

- Elaborate the importance of isolation in an amplifier.

The output of an amplifier can pick up stray signal via conductors connected to it, and this signal can make it way into modules connected before the amplifier. High isolation is needed (typically > 20 dB) to suppress the stray signal. Moreover, an amplifier with high isolation is more stable, it is less susceptible to oscillation.

An ideal amplifier would have infinite reverse isolation; no signal would be transmitted from the output back to the input. However, reflected signals can pass through the amplifier in the reverse direction. This unwanted reverse transmission can cause the reflected signals to interfere with the desired fundamental signal flowing in the forward direction. Therefore, reverse isolation is important to quantify.

Reflection Measurements

- Use the following settings to determine the input return loss of the PA:

Format 	:  Log magnitude

Meas		:  S11

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S11}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the minimum input return loss and the frequency at which it occurs?

Min return loss = 9.5 dB at 700 MHz

- Find the maximum input return loss and the frequency at which it occurs?

Max return loss = 28 dB at 880 MHz

- Find the input return loss at 868 MHz.

Return loss @868 MHz = 12 dB

[Note: Use the marker function to search for maximum and minimum]

- Use the following settings to determine the output return loss of PA:

Format 	:  Log magnitude

Meas		:  S22

Scale		:  Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Format] > {Log Mag}

Measure: [Meas] > {S22}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Find the minimum output return loss and the frequency at which it occurs?

Min return loss = 10.5 dB at 1000 MHz

- Find the maximum output return loss and the frequency at which it occurs?

Max return loss = 16 dB at 890.5 MHz

- Find the output return loss at 868 MHz.

Return loss @868 MHz = 15 dB

[Note: Use the marker function to search for maximum and minimum]

Gain compression measurement with power sweep

- Perform a power sweep at 868 MHz by setting the following:

Sweep Setup 	:  CW Frequency to 868 MHZ

:  Sweep Type to Power Sweep

Power		:  Power Range (–15dBm to 0dBm)

Start		:  –15 dBm

Stop		:  0 dBm

Meas		:  S21

Scale		: Autoscale

E5061A Setting: “[ ]”: Hardkey; “{ }”: Softkey

Sweep Type: [Sweep Setup] > {Sweep Type} > {Power Sweep}

CW Frequency: {Power} > {CW Frequency} > [868] > [M/μ]

Power Ranges: {Power Ranges} > {-15 to 0}

Measure: [Meas] > {S21}

Scale: [Scale] > {Auto Scale}

OBSERVATIONS/DATA RECORDING

- Define and elaborate the meaning of 1 dB compression point.

An amplifier has a region of linear gain, where the gain is independent of the power level. This gain is commonly referred to as small signal gain. As the input power is increased to a level that causes the amplifier to approach saturation, the gain will decrease, resulting in a large signal response. The 1 dB gain compression is defined as the input power level where the amplifier gain drops 1 dB relative to the small signal gain.

- Plot the gain vs power sweep graph. Use marker to locate the P1 dB point.

- What is the VNA source power setting at 1 dB compression point?

P1 dB (input) = −5.282 dBm

- What is the measured gain at 1 dB compression?

Gain @P1 dB = 14 dB

- Is the above P1dB measured refers to input or output?

Input P1dB.

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements”, Keysight Technologies.

- Application Note 1287-4, “Network Analyzer Measurements: Filter and Amplifier Examples”, Keysight Technologies.

- Product Note 8753-1, “RF Component Measurements: Amplifier Measurements Using the Keysight 8753 Network Analyzer”, Keysight Technologies.

- Thomas H. Lee, “Planar Microwave Engineering”, Cambridge University Press, 2004.