ME1000 RF Circuit Design

Lab 6

(Solution)

Antenna Reflection Measurement with

RF Analyzer

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

Objective

- To characterize an antenna with reflection measurement using an RF analyzer (RFA)

Equipment Required

- Keysight N9912A FieldFox RF Analyzer

Accessories Required

- 2 x 868 MHz Antenna

# Introduction

RF Analyzer

RF OUT

RF IN

Basic Equipment Setup

RF Analyzer

RF OUT

RF IN

Antenna-Under-Measurement (Flat)

Antenna-Under-Measurement (Vertical)

(b)

Figure 1 – General Equipment Configuration for Antenna Measurement

(a)

The RF OUT port of the RF analyzer (RFA) generates a sinusoidal wave. The wave interacts with the device-under-test (DUT) and the result is captured by the RF IN port of the RFA. Therefore, the RF OUT port is always connected to the input of the DUT and the RF IN is connected to the output of the DUT.

In this lab, you need to set the RFA to the Network Analyzer (NA) mode. When performing a transmission, or an S21 measurement, carry out the calibration procedures from Lab 1 with the SMA coaxial cables connected to both the RF OUT port and RF IN port of the RFA before you proceed. The cable and connectors on the RF OUT port of the RFA should be as short as possible and of high quality.

Note:

If your RFA comes with Option 111 (‘QuickCal’), you will be able to perform external calibration on the RF OUT port for reflection or an S11 measurement. Refer to the Keysight N9912A User’s Guide for the detailed procedure.

When viewing signals, the [Freq/Dist], [Scale/Amptd], and [Marker] hardkeys are the most helpful. When the [Scale/Amptd] hardkey is pressed, the {Autoscale} option can be quite handy, but you will rarely use it as you get more comfortable with manipulating the display.

Note: 
Use the marker ([Marker] hardkey) to record your readings. The marker is used to obtain a reading at a particular frequency or amplitude. For simplicity, you may wish to use the Marker Function or Marker Search option to search for maximum and minimum points.

# Reflection Measurements

- Set the RFA (N9912A) to the NA mode for S11 measurement.

- Connect the antenna to the N9912A as shown in Figure 1(a).

- Use the following settings to determine the insertion loss of the antenna:

Format 	:  Log magnitude

Measurement	:  S11

Scale		:  Autoscale

N9912A setting: “[ ]”: Hardkey; “{ }”: Softkey

Set to the NA mode for S21 measurement: [Mode] > {NA} > {S11}

Set the display to the Log magnitude format: [Meas Setup] > {Format} > {LogMag}

Set the Start Frequency to 500 MHz: [Freq/Dist] > {Start} > [500] > {MHz}

Set the Stop Frequency to 1.5 GHz: [Freq/Dist] > {Stop} > [1.5] > {GHz}

Set the output power level to LOW: [Meas Setup 4] > {Output Power} > {Low}

Scale: [Scale/Amptd] > {Autoscale}

Exercises

Observations/data recording – Flat position

- Plot the return loss, S11 vs. frequency. Use the marker function to determine the minimum point.

Minimum return loss, S11 = −25.8 dB at 825 MHz, −11.4 dB at

Frequency (GHz)

Return Loss, S11 (dB)

868 MHz

- From the minimum return loss observed, describe the frequency/frequencies tuned by the antenna.

The frequency/frequencies tuned by the antenna is of the lowest return loss (S11).

- Using c = f , estimate the required length of the wired antenna on the tuned frequency. Does it correlate with the measured return loss? Explain your answer.

The might be some discrepancy due to the reflection observed during measurement of the return loss.

- Change the settings to display the standing wave ratio (SWR).

Format 	:  VSWR

N9912A setting: “[ ]”: Hardkey; “{ }”: Softkey

Format: [Meas Setup] > {Format} > {VSWR}

Exercise

Observations/data recording – Flat position

- What is the standing wave ratio at the tuned frequency?

SWR at tuned frequency = 1: 1.07

- Change the settings to display the Smith Chart for input impedance measurement.

- From the Smith Chart, you can determine the magnitude and phase of the reflection coefficient.
From these values, you can convert the reflection coefficient into the complex form, and finally convert the reflection coefficient into the input impedance Zin, via Zin=1+Γ111−Γ11⋅Zo , where Zo is 50 

Format 	:  Smith Chart

N9912A setting: “[ ]”: Hardkey ; “{ }”: Softkey

Format: [Meas Setup] > {Format} > {Smith}

Exercises

Observations/data recording – Flat position

- What is the impedance at the tuned frequency?

Impedance at tuned frequency = 45.5 + j 0.4 Ω

- What is the ideal impedance at the tuned frequency?

Ideal impedance at pass-band = 50 Ω

- Bend the antenna to the vertical position as shown in Figure 1(b). Repeat steps 2 to 5, and record your observations.

Exercises

Observations/data recording – Vertical position

- Plot the minimum return loss, S11 vs. frequency. Use the marker function to determine the minimum point.

Minimum return loss, S11 = −17.5 dB at 825 MHz, −8.5 dB at 868 MHz

Frequency (GHz)

Return Loss, S11 (dB)

- From the minimum return loss observed, describe the frequency/frequencies tuned by the antenna.

The frequency/frequencies tuned by the antenna is of the lowest return loss (S11).

- Are the results similar with the previous measurement where the antenna is in the flat position? Comment, and explain your answer.

No. This is because the strength of electromagnetic fields received in these two positions is different.

- What is the standing wave ratio at the tuned frequency?

SWR at the tuned frequency = 1: 1.26

- What is the impedance at the tuned frequency?

The impedance at the tuned frequency = 41.4 + j 7.8 Ω

- What is the ideal impedance at passband?

The ideal impedance at passband = 50 Ω

References

- Application Note 1287-1, “Understanding the Fundamental Principles of Vector Network Analysis”, Keysight Technologies.

- Application Note 1287-2, “Exploring the Architectures of Network Analyzers”, Keysight Technologies.

- Application Note 1287-3, “Applying Error Correction to Network Analyzer Measurements”, Keysight Technologies.