ME1000 RF Circuit Design

Assignment 4

RF Amplifier Design

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

Objectives

- To design and develop a single-stage, small signal RF amplifier

- To measure the performance of the constructed RF amplifier

Equipment Required

- ME1000 RF Transceiver Kit (Transmitter Unit)

- ME1000 RF Transceiver Kit (Receiver Unit)

- Vector Network Analyzer or Keysight N9320B 3 GHz RF Spectrum Analyzer

- E8975L Advanced Design System (ADS) Software

##### Accessories Required

- 2 × SMA(m)-to-SMA(m) coaxial cable

- 9 × SMA(m)-to-SMA(m) jumper cable

- 2 × USB cables

- 2 × antennas

- A PC running Microsoft® Windows XP/Vista® with a minimum of 512 MB of RAM and USB ports, 
pre-installed with the RF Trainer Control Panel software

- Measurement Task

With the help of the Advanced Design System (ADS) software, design a single-stage small-signal amplifier with the following parameters:

- Power supply: 3 V to 5 V

- Operating frequency: 700 MHz to 900 MHz

- Power gain within the operating frequency range: 10 dB minimum

- Source impedance: 50 Ω

- Load impedance: 50 Ω

You can refer to the procedures and schematics in Chapter 8 of the ME1000 RF Circuit Design teaching slides. Use the ADS software to analyze your schematics and to ensure that it works.

You can use the ADS software, or other PCB layout software of your choice, to draw the printed circuit board (PCB) layout. Fabricate the PCB and build the amplifier. Remember to include the SMA connectors on the input and output terminals. Verify that your amplifier is working using a vector network analyzer (VNA) or a spectrum analyzer (SA) with a tracking generator (TG). If you use the VNA, measure the small-signal S-parameters of your amplifier. From the S-parameters, calculate the small-signal power gain GP, GA, and GT of the amplifier. If you use the SA, only the magnitude response of the output over input can be measured. You can also perform a power sweep measurement using either the VNA or SA to determine the 1 dB gain compression point. If you have access to a noise figure meter, a noise figure measurement can also be carried out.

Connect the TX and RX units as a transmit/receive system as shown in Figure 1. Use your amplifier in place of the low-noise amplifier (LNA) in the RX unit. Verify if you can detect a radiowave signal at 868 MHz. Briefly discuss your observation on how your amplifier fares compared to the LNA on the RX unit.

50 MHz

–15 dBm

818 MHz

818 MHz

868 MHz

50 MHz

Function

Generator

VNA or Spectrum

Analyzer

TX Unit

RX Unit

Figure 1 – Connecting the TX and RX Units as a Transmit/Receive System