ME1000 RF Circuit Design

Assignment 3

RF Bandpass Filter Design

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

Objectives

- To design an RF bandpass filter using ADS software

- To construct an RF bandpass filter

- To measure the performance of the constructed RF bandpass filter

Equipment Required

- ME1000 RF Transceiver Kit (Transmitter Unit)

- ME1000 RF Transceiver Kit (Receiver Unit)

- Vector Network Analyzer or Keysight N9320B 3 GHz RF Spectrum Analyzer

- E8975L Advanced Design System (ADS) Software

##### Accessories Required

- 2 × SMA(m)-to-SMA(m) coaxial cable

- 9 × SMA(m)-to-SMA(m) jumper cable

- 2 × USB cables

- 2 × antennas

- A PC running Microsoft® Windows XP/Vista® with a minimum of 512 MB of RAM and USB ports, 
pre-installed with the RF Trainer Control Panel software

- Measurement Task

With the help of the Advanced Design System (ADS) software, design a 4th order bandpass filter with the following parameters:

- Passband (3 dB): 45 MHz to 55 MHz

- Insertion loss in the passband: Not more than 5 dB

- Return loss in the passband: 10 dB or larger

- Filter type: Either Butterworth or Chebyshev (0.3 dB ripple)

- Source impedance: 50 Ω

- Load impedance: 50 Ω

You can refer to the procedures and schematics in Chapter 3 of the ME1000 RF Circuit Design teaching slides. Use the ADS software to analyze your schematic and to ensure that it works.

You can use the ADS software, or other PCB layout software of your choice, to draw the printed circuit board (PCB) layout. Fabricate the PCB and build the bandpass filter. Remember to include the SMA connectors on the input and output terminals. Verify that your amplifier is working using a vector network analyzer (VNA) or a spectrum analyzer (SA) with a tracking generator (TG). If you use the VNA, measure the small-signal S-parameters. If you use the SA, only the magnitude response of the output over input can be measured.

Connect the TX and RX units as a transmit/receive system as shown in Figure 1. Use your filter in place of the IF bandpass filter in the RX unit. Verify if you can detect a radiowave signal at 868 MHz. Briefly discuss your observation on how your filter fares compared to the IF bandpass filter on the RX unit.

50 MHz

–15 dBm

818 MHz

818 MHz

868 MHz

50 MHz

Function

Generator

VNA or Spectrum

Analyzer

TX Unit

RX Unit

Figure 1 – Connecting the TX and RX Units as a Transmit/Receive System