ME1000 RF Circuit Design

Assignment 2

Maximum Operating Distance Measurement

Using Oscilloscope

This courseware product contains scholarly and technical information and is protected by copyright laws and international treaties. No part of this publication may be reproduced by any means, be it transmitted, transcribed, photocopied, stored in a retrieval system, or translated into any language in any form, without the prior written permission of Acehub Vista Sdn. Bhd.

The use of the courseware product and all other products developed and/or distributed by Acehub Vista Sdn. Bhd. are subject to the applicable License Agreement.

For further information, see the Courseware Product License Agreement.

Objectives

- To understand and measure the signal-to-noise ratio (SNR) of an RF receiver using an oscilloscope

- To determine the maximum operating distance between a transmitter and a receiver

Equipment Required

- ME1000 RF Transceiver Kit (Transmitter Unit)

- ME1000 RF Transceiver Kit (Receiver Unit)

- Keysight N9320B 3 GHz RF Spectrum Analyzer

- Keysight DSO6012A/DSO7012A Oscilloscope, 100 MHz

##### Accessories Required

- 2 × SMA(m)-to-SMA(m) coaxial cable

- 9 × SMA(m)-to-SMA(m) jumper cable

- 2 × USB cables

- 2 × antennas

- A PC running Microsoft® Windows XP/Vista® with a minimum of 512 MB of RAM and USB ports, 
pre-installed with the RF Trainer Control Panel software

- Measurement Task

In place of the spectrum analyzer (see the ME1000 RF Circuit Design Quick Start Guide), you can connect the IF output of the RX transceiver unit to an oscilloscope with a bandwidth of at least 
100 MHz — this setup shown in Figure 1. Make sure that the correct probe is used to connect the IF output to the oscilloscope. Typically, a 10X probe with a bandwidth of at least 200 MHz would be required.

Using AC coupling for the oscilloscope, set the horizontal scale to between 25 ns to 100 ns per division and the vertical scale to between 10 mV to 50 mV per division. When a 50 MHz IF signal is injected into the TX unit, you should be able to observe a sinusoidal waveform of 50 MHz appearing on the oscilloscope display. Conversely, when the IF signal is turned off, the noise signal will appear on the display of the oscilloscope.

Measure the root-mean-square (RMS) value of the noise voltage using the oscilloscope (the modern digital oscilloscope has this capability). From this you can measure the signal-to-noise ratio (SNR) at the IF output of the RX unit. Assuming an SNR of at least 2 is needed to distinguish between the signal and noise, determine the maximum separation allowable between the TX and RX units.

50 MHz

–15 dBm

818 MHz

818 MHz

868 MHz

Function

Generator

Digital Oscilloscope

TX Unit

RX Unit

Figure 1 – Connecting the TX and RX Units as a Transmit/Receive System